Department of Homeland Security

Customs and Border Protection

And

Domestic Nuclear Detection Office

N.25 Radiation & Nuclear Detection Message Protocol

Information Exchange Package Documentation

Volume 8. Device Configuration and Health Status Message

Document Number 500-IECBP-107350 Volume 8v1.1.40

14 February 2009

Change History

The following table indicates the changes that were made to this document since its last release. Note: version number is formed to correspond to the IEPD version number as follow:    MajorVersionNumber.MinorVersionNumber.BuildNumber.

| New Version | Modified By | Section | Page | Change Made |
| --- | --- | --- | --- | --- |
| 1.0.39 | W. R. Wright | All | All | Initial Release |
| 1.1.40 | CBP TASPO and W. R. Wright | All | All | Minor administrative updates |

Table of Contents

Change History	2

1	Overview	4

1.1	Purpose	4

1.2	Introduction	4

1.3	IEPD Artifacts	4

1.4	Reference Documents	5

2	Business Scenario	5

3	Business Requirements	5

4	Business Processes and Rules	5

5	Implementer Notes	5

5.1	General	5

5.2	Common	6

5.2.1	Request – Response Message Mapping	6

5.2.2	Message Content Header	7

5.2.3	Coordinated Universal Time	7

5.3	DeviceConfigurationParameterReport Details	7

5.4	DeviceAssemblyReport Details	8

5.5	DeviceCommConnectionsReport Details	8

5.6	DeviceHealthStatusReport Details	9

5.7	DeviceCommHealthStatusReport Details	9

5.8	DeviceUpgradeReport Details	10

Appendix A.	Schema Outline	11

List of Tables

Table 1.  Reference Documents	5

Table 2. Request – Response Message Mapping	6

N.25 Device Configuration and Health Status Message

# Overview

## Purpose

This document provides details of the N.25 Device Configuration and Health Status Message. The intent is to assist implementers understanding of the data requirements for this message type.

## Introduction

This message group provides an exchange for transmitting data about operationally deployed detection devices regarding their configuration and health status.  Most messages in the group are responses to request messages defined in the RequestMessage group (see Master Document Vol. 7). One message is asynchronously sent from the detection site.

The message schema provides the following messages for reporting on a device’s configuration and health status:

- DeviceConfigurationParameterReport	--  Provides a report on device configuration parameters for the requested device(s).

- DeviceAssemblyReport	--  Provides a report describing the assembly level structure of the requested device(s).

- DeviceCommConnectionsReport	--  Provides a report regarding the communication connections of the requested device(s).

- DeviceHealthStatusReport	--  Provides a report on the health status of the requested device(s).

- DeviceCommHealthStatusReport	--  Provides a report regarding the status of the communication connections of the requested device(s).

- DeviceUpgradeReport	--  Provides a report regarding a device(s) upgrade that has occurred.

Detailed notes on each of these messages are provided in later sections of this document.

## IEPD Artifacts

The N.25 IEPD artifacts for this message are:

- This document

- NIEM compliant XML exchange schemas (unconstrained and constrained)

- Component Mapping Template

- Visual model of the message schema

- Sample Messages

The detailed list of the digital data files that comprise the N.25 IEPD artifacts for this message is contained in the IEPD Catalog.

## Reference Documents

The following table lists the reference documents for this message.

Table 1.  Reference Documents

| Document Type | Date | Document Title |
| --- | --- | --- |
| Master Document |  | Master Document Vol. 7 – Request Message |

# Business Scenario

The DeviceHealthAndConfigMessage message represents the configuration, health status and threat level settings of the instrumentation and devices at a detection site.  There are two business scenarios for this message type: (1) a request message for device configuration or health status has been received, and a response is to be prepared and sent, and (2) a device detects that it has been changed in some manner, so it asynchronously transmits a report of its current configuration and status.

# Business Requirements

The basic business requirement for this message is to transmit device configuration and health status data from detection sites where they are deployed.  This includes both fixed and mobile sites.

# Business Processes and Rules

To understand the general flow of the Device Configuration and Health Status messages, refer to the business process described in section 4 of Volume 7, Request Message.  This provides a detailed description of the process flow for the SiteInfo Request-Response message pair.

# Implementer Notes

## General

1. The schema outline provided in Appendix A provides a structured list of the desired schema elements and attributes.  Examination of the schema with an xml tool such as XML Spy or Stylus Studio will reveal more elements/attributes than appear in this outline. This is due to type inheritance that occurs with imports from referenced schema.  There is no requirement to implement any elements/attributes that do not appear in the schema outline in this document, unless specifically addressed in the detailed notes in this section.

2. The names in the schema outline in Appendix A are the xml tagged element names that will appear in a message instance. A name enclosed in parentheses following the element name is an attribute.

3. Indentation is used in the schema outline to show parent-child nesting.

4. The schema outline includes element cardinality rules on the same line with the element, enclosed in brackets. The values and meanings are:

[1,1]  The element is required and there may be only one instance.

[0,1] The element is optional and there may be zero or one instance.

[0 or many]  The element is optional and there may be zero, one or many instances.

In some cases if an optional element is used, then a child element may be required. The rules for these are specified in the details in this section.

5. The descriptions explaining each element and attribute is contained in the schema xsd file in a ‘xsd:documentation’ statement immediately following the declaration of the element, attribute or type.  The information in this section 5 is supplementary to the documentation in the schema xsd file.

6. Two schema files are provided for each message: unconstrained and constrained. They are denoted by the schema filename suffix ‘-x’ and ‘-xc’ for unconstrained and constrained respectively.  The difference between the two is that the constrained schema includes cardinality constraints and other types of constraints, such as max and min values, that may be applicable. The constrained schema is used for the messages defined in this N.25 IEPD.  The unconstrained schema is provided as part of the general NIEM concept of share and reuse of data definitions by other domains and IEPDs.

## Common

### Request – Response Message Mapping

Appendix C of the N.25 IEPD Master Document Volume 7 – Request Message,  provides a table of the parameter settings that define which configuration and health status report is requested and the scope of the request (one, multiple, or all devices at the site). The following table identifies the report to be transmitted in response to a valid Device Configuration and Health Status Request message, based on the value of the DeviceAssociationKindCode and ConfigStatusKindCode elements in the request message.

Table 2. Request – Response Message Mapping

| Report | DeviceAssociationKindCode | ConfigStatus KindCode |
| --- | --- | --- |
| Device Assembly Connections Report | Subassembly | Configuration |
| Device Assembly Connections Report | Component | Configuration |
| Device Comm Connections Report | Wired Comm | Configuration |
| Device Comm Connections Report | Wireless Comm | Configuration |
| Device Comm Connections Report | Other Comm | Configuration |
| Device Comm Connections Report | Comm | Configuration |
| Device Health Status Report | Subassembly | Status |
| Device Comm Health Status Report | Wired Comm | Status |
| Device Comm Health Status Report | Wireless Comm | Status |
| Device Comm Health Status Report | Other Comm | Status |
| Device Comm Health Status Report | Comm | Status |
| Device Config Parameter Report | Subassembly | Parameter |
| Device Config Parameter Report | Component | Parameter |

Any other combination of values of DeviceAssociationKindCode and ConfigStatusKindCode elements in the request message is an error and will not result in a message from this message group.

### Message Content Header

All messages have a common header.  The following provide special notes on the elements of the header:

<MessageOrigin> -  To identify the message origin, at least one of <FacilityID>, <FacilityName>, or <OrganizationIdentification>, <OrganizationName> must be provided.

<MessageDestination> - At least one destination is required.A message may be sent to multiple destinations by including this element for each destination with at least one of  <FacilityID>, <FacilityName>, or <OrganizationIdentification>, <OrganizationName>.

### Coordinated Universal Time

Coordinated Universal Time (UTC) shall be used in all date and time elements.  XML standards use the dateTime format based on the ISO 8601 dateTime [YYYY-MM-DDTHH:MM:SSZ] where the “Z” is appended to indicate Universal Time (UTC). Note that in the event of times/durations involving fractions of a second, the separator between seconds and fractions of seconds may be a comma or the full stop, eg. “.”.  The number of fraction digits shall not exceed three (3).

The information in each of the following sections supplements the information in the schema for the identified elements.  If an element is not discussed, this means that there is no supplementing information.

## DeviceConfigurationParameterReport Details

This submessage provides a report on device configuration parameters for the requested device(s).  This message is provided in response to the Device Configuration and Health Status Request message (see Table 2 above).

< DeviceConfigurationParameter> - This element is repeated for each configuration parameter that the device can provide.  The substitution group allows for a choice of type of parameter value.  If the parameter name is specific to the device vendor, the name element tag should be <DeviceParameterVendorName>.

## DeviceAssemblyReport Details

This submessage provides a report describing the assembly level structure of the requested device(s). This message is provided in response to the Device Configuration and Health Status Request message (see Table 2 above).

The following describes the expected values of each element specific to this report.

<DeviceAssemblyConnectionData> - Repeated for each assembly type connection for the device identified by the element <RadEncounterDeviceID>.

<EncounterDeviceCategoryLevelCode> - The value of this element defines whether the device identified in <ToDeviceID> is higher or lower in the assembly tree than <RadEncounterDeviceID>.

- If the value is ‘system’, then <ToDeviceID> is lower in the tree.

- If the value is ‘device’, then <ToDeviceID> is higher in the tree.

- If the value is ‘component’, then <ToDeviceID> is higher in the tree and <RadEncounterDeviceID> is at the bottom of the assembly tree.

- Any other value is invalid.

<DeviceAssociationKindCode> - Mandatory element.  The value must be either ‘Subassembly’ or ‘Component’, with meanings as described in the code list table. Any other value is invalid.

<SimplexIndicator> - This element is not permitted in this report.  It applies only to communications connections.

<ToDeviceID> - This is the identifier of the assembly/component higher or lower in the assembly tree from the device that is the subject of the report.

<DeviceAssociationStatusCode> - Typically for this report this element is not required. However, is a status code is meaningful it may be provided.

## DeviceCommConnectionsReport Details

This submessage provides a report regarding the communication connections of the requested device(s). This message is provided in response to the Device Configuration and Health Status Request message (see Table 2 above).

The following describes the expected values of each element specific to this report.

<DeviceCommConnectionData> - Repeated for each communications type connection for the device identified by the element <RadEncounterDeviceID>.

<EncounterDeviceCategoryLevelCode> - The value of this element defines whether the device identified in <ToDeviceID> is higher or lower in the communication connections tree than <RadEncounterDeviceID>.  Typically this element may not be meaningful and may be omitted. If used, the following are the meanings of the codes in the codelist.

- If the value is ‘system’, then <ToDeviceID> is lower in the tree.

- If the value is ‘device’, then <ToDeviceID> is higher in the tree.

- If the value is ‘component’, then <ToDeviceID> is higher in the tree and <RadEncounterDeviceID> is at the bottom of the tree.

- Any other value is invalid.

<DeviceAssociationKindCode> - Mandatory element.  The value must be either ‘Comm’, Wired Comm’, ‘Wireless Comm’, or ‘Other Comm’, with meanings as described in the code list table. Any other value is invalid.

<SimplexIndicator> - True if the comm link is simplex; false otherwise.

<ToDeviceID> - Mandatory element. This is the identifier of the device to which <RadEncounterDeviceID> is connected.

<DeviceAssociationStatusCode> - Typically for this report this element is not required. However, is a status code is meaningful it may be provided.

## DeviceHealthStatusReport Details

This submessage provides a report on the health status of the requested device(s). This message is provided in response to the Device Configuration and Health Status Request message (see Table 2 above).

The following describes the expected values of each element specific to this report.

< DeviceHealthStatusData> - Repeated for each device whose health status is being reported. The device is identified by the element <RadEncounterDeviceID>.

<EncounterDeviceCategoryCode> -  Identifies the category of encounter device.

<EncounterDeviceStatus  -  Mandatory. Header tag for the following status data.

<EncounterDeviceStatusDateTime  - Mandatory. Time the device reported status.

<OperationalIndicator> - True if the device is operational; false otherwise.

<EncounterDeviceHealthStatusCode – Additional description of health status if available and meaningful.

<EncounterDeviceStatusParameters> – If health status parameters are available, this element provides a means to report health status parameters and their values.

<ParameterNames>  - A white-space delimited list of names. Only required if <EncounterDeviceStatusParameters> is used in the message.

<ParameterValues> - A white-space delimited list of values, with the values represented as strings. Only required if <EncounterDeviceStatusParameters> is used in the message. The order of the values must match the order of the names.

## DeviceCommHealthStatusReport Details

This submessage provides a report regarding the status of the communication connections of the requested device(s). This message is provided in response to the Device Configuration and Health Status Request message (see Table 2 above).

The following describes the expected values of each element specific to this report.

< DeviceCommHealthStatusData> - Repeated for each device whose communications health status is being reported. The device is identified by the element <RadEncounterDeviceID>.

<DeviceConnection> - Repeated for each comm connection being reported for a device.

<EncounterDeviceCategoryLevelCode> - This element is not relevant in this report and should be omitted.

<DeviceAssociationKindCode> - This element is not relevant in this report and should be omitted.

<SimplexIndicator> - This element is not relevant in this report and should be omitted.

<ToDeviceID> - Mandatory. Provides identifier of the device at the other end of the comm link.

<DeviceAssociationStatusCode> - Mandatory.  Report status of the comm connection with the appropriate code from the codelist.

## DeviceUpgradeReport Details

This submessage provides a report regarding a device(s) upgrade that has occurred.  This message is automatically sent by the site controller when it detects that a device has been upgraded/changed in some manner.

To the extent the device is capable of providing the data, all elements should be used to include the device configuration and status parameters.

# Schema Outline

DeviceConfigAndHealthStatusMessage [1,1]

MessageContentHeader [1,1]

MessageID [1,1]

MessageCreationDateTime [1,1]

MessageDispatchDateTime [1,1]

MessageKindCode [1,1]

MessagePriorityCode [0,1]

MessageOrigin [1,1]

Choice of two of the following

FacilityID [0,1]

FacilityName [0,1]

OrganizationIdentification [0,1]

OrganizationName [0,1]

MessageDestination [1,many]

Choice of two of the following

FacilityID [0,1]

FacilityName [0,1]

OrganizationIdentification [0,1]

OrganizationName [0,1]

MessageVersionText [0,1]

RemarkText [0,1]

MessageContent [1,1]

DeviceConfigurationParameterReport [0,1]

ReportDateTime [1,1]

ActivityName [0,1]

CredentialsAuthenticatedCode [1,1]

RequestData [1,1]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

DeviceConfigurationParameterData (radEncounterDeviceOperatingCode) [0,many]

DeviceResponseDateTime [0,1]

EncounterDeviceID [0,1]

EncounterDeviceCategoryCode [0,1]

DeviceConfigurationParameter (unitsText) [0,many]

DeviceParameterName [0,1]

DeviceParameterVendorName [0,1]

DeviceParameterValue [1,1]

Choice of one or two of the following

DeviceParameterValueText [0,1]

DeviceParameterPointValue [0,1]

DeviceParameterMaxValue [0,1]

DeviceParameterMinValue [0,1]

DeviceParameterValueIndicator [0,1]

DeviceParameterValueListText [0,1]

DeviceParameterValueListName [0,1]

DeviceAssemblyReport [0,1]

ReportDateTime [1,1]

ActivityName [0,1]

CredentialsAuthenticatedCode [1,1]

RequestData [1,1]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

DeviceAssemblyConnectionData (radEncounterDeviceOperatingCode) [0,many]

DeviceResponseDateTime [0,1]

EncounterDeviceID [0,1]

EncounterDeviceCategoryCode [0,1]

DeviceConnection [0,many]

EncounterDeviceCategoryLevelCode [0,1]

DeviceAssociationKindCode [0,1]

SimplexIndicator [0,1]

ToDeviceID [1,1]

DeviceAssociationStatusCode [0,1]

DeviceCommConnectionsReport [0,1]

ReportDateTime [1,1]

ActivityName [0,1]

CredentialsAuthenticatedCode [1,1]

RequestData [1,1]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

DeviceCommConnectionData (radEncounterDeviceOperatingCode) [0,many]

DeviceResponseDateTime [0,1]

EncounterDeviceID [0,1]

EncounterDeviceCategoryCode [0,1]

DeviceConnection [0,many]

EncounterDeviceCategoryLevelCode [0,1]

DeviceAssociationKindCode [0,1]

SimplexIndicator [0,1]

ToDeviceID [1,1]

DeviceAssociationStatusCode [0,1]

DeviceHealthStatusReport [0,1]

ReportDateTime [1,1]

ActivityName [0,1]

CredentialsAuthenticatedCode [1,1]

RequestData [1,1]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

DeviceHealthStatusData (radEncounterDeviceOperatingCode) [0,many]

DeviceResponseDateTime [0,1]

EncounterDeviceID [0,1]

EncounterDeviceCategoryCode [0,1]

EncounterDeviceStatus [1,1]

EncounterDeviceStatusDateTime [1,1]

OperationalIndicator [1,1]

EncounterDeviceHealthStatusCode [0,1]

EncounterDeviceStatusParameters [0,many]

ParameterNames [1,1]

ParameterValues [1,1]

DeviceCommHealthStatusReport [0,1]

ReportDateTime [1,1]

ActivityName [0,1]

CredentialsAuthenticatedCode [1,1]

RequestData [1,1]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

DeviceCommHealthStatusData (radEncounterDeviceOperatingCode) [0,many]

DeviceResponseDateTime [0,1]

EncounterDeviceID [0,1]

EncounterDeviceCategoryCode [0,1]

DeviceConnection [0,many]

EncounterDeviceCategoryLevelCode [0,1]

DeviceAssociationKindCode [0,1]

SimplexIndicator [0,1]

ToDeviceID [1,1]

DeviceAssociationStatusCode [0,1]

DeviceUpgradeReport [0,many]

ReportDateTime [1,1]

ActivityName [0,1]

CredentialsAuthenticatedCode [1,1]

DeviceUpgradeData [0,many]

DeviceUpgradeDateTime [1,1]

DeviceInformation (radEncounterDeviceModeCode, radEncounterDeviceOperatingCode) [1,1]

RemarkText [0,1]

EncounterDeviceCategoryCode [0,1]

EncounterDeviceCategoryLevelCode [0,1]

Choice of one of the following

EncounterDeviceID [0,1]

EncounterDeviceMfgID [0,1]

EncounterDeviceAssetID [0,1]

ItemMakeName [0,1]

ItemModelName [1,1]

EncounterDeviceVersion [0,1]

VersionKindCode [0,1]

CurrentVersionID [0,1]

PriorVersionID [0,1]

EncounterDeviceStatus [0,1]

EncounterDeviceStatusDateTime [1,1]

OperationalIndicator [1,1]

EncounterDeviceHealthStatusCode [0,1]

EncounterDeviceStatusParameters [0,many]

ParameterNames [1,1]

ParameterValues [1,1]

EncounterDeviceConfiguration [0,1]

DeviceParameter (parameterUnitsText) [0,many]

DeviceParameterName [0,1]

DeviceParameterVendorName [0,1]

DeviceParameterSettableIndicator [0,1]

DeviceParameterStatusIndicator [0,1]

Choice of one or two of the following

DeviceParameterValueText [0,1]

DeviceParameterPointValue [0,1]

DeviceParameterMaxValue [0,1]

DeviceParameterMinValue [0,1]

DeviceParameterValueIndicator [0,1]

DeviceParameterValueListText [0,1]

DeviceParameterValueListName [0,1]

EncounterDeviceProbe [0,1]

EncounterDeviceProbeKindCode [0,1]

EncounterDeviceProbeModelName [0,1]

EncounterDeviceProbeID [0,1]

EncounterDeviceQualityControl [1,1]

RemarkText [0,1]

InspectionDate [1,1]

InServiceIndicator [1,1]