Department of Homeland Security

Customs and Border Protection

And

Domestic Nuclear Detection Office

N.25 Radiation & Nuclear Detection Message Protocol

Information Exchange Package Documentation

Volume 7. Request Message

Document Number 500-IECBP-107350 Volume 7v1.1.40

14 February 2009

Change History

The following table indicates the changes that were made to this document since its last release. Note: version number is formed to correspond to the IEPD version number as follow:    MajorVersionNumber.MinorVersionNumber.BuildNumber.

| New Version | Modified By | Section | Page | Change Made |
| --- | --- | --- | --- | --- |
| 1.0.39 | W. R. Wright | All | All | Initial Release |
| 1.1.40 | CBP TASPO and W. R. Wright | All | All | Change RadNucCase to CBRNCase. Other minor adminstrative updates. |

Table of Contents

Change History	2

1	Overview	5

1.1	Purpose	5

1.2	Introduction	5

1.3	IEPD Artifacts	5

1.4	Reference Documents	6

2	Business Scenario	6

3	Business Requirements	6

4	Business Processes and Rules	6

5	Implementer Notes	12

5.1	General	12

5.2	Common	12

5.3	RequestDeviceConfigOrHealthStatus Details	13

5.3.1	General	13

5.3.2	Request a Device Assembly Connections Report	13

5.3.3	Request a Device Comm Connections Report	15

5.3.4	Request a Device Health Status Report	15

5.3.5	Request a Device Comm Health Status Report	16

5.3.6	Request a Device Config Parameter Report	16

5.3.7	Request a Device Comm Connection Parameter Report	17

5.4	RequestScanData Details	18

5.5	RequestContainerListSummary Details	18

5.6	RequestSiteConfigData Details	19

5.7	TraversalSummaryRequest Details	19

5.8	CBRNCaseRequest	19

5.9	ReachbackRequest Details	22

Appendix A.	Schema Outline	24

List of Figures

Figure 1.  SiteInfo Request-Response Process	11

List of Tables

Table 1.  Reference Documents	6

Table 2. Site Info Request-Response Process and Rules	7

Table 3. Element Values to Request Device Assembly Connections Report	14

Table 4. DeviceAssociationKindCode Table	14

Table 5. Element Values to Request Device Comm Connections Report	15

Table 6. Element Values to Request a Device Health Status Report	15

Table 7. Element Values to Request a Device Comm Health Status Report	16

Table 8. Element Values to Request a Device Configuration Parameter Report	16

Table 9. DeviceParameterValue Choices	17

Table 10. Element Values to Request a Device Comm Parameter Report	18

Table 11. Container List Scope Codes	19

Table 12. Case Request Kinds	20

Table 13. Case Request Required Data	20

Table 14. Case Status Codes	21

Table 15. Case Threat Level Codes	21

Table 16. Case RequestScopeCode	22

Table 17. Reachback Request Code Useage	22

N.25 Request Message

# Overview

## Purpose

This document provides details of the N.25 Request Message. The intent is to assist implementers understanding of the data requirements for this message type.

## Introduction

This message group provides exchanges for requesting data from a detection site or assistance from reachback analysts.  The following request messages are included in this exchange schema.

RequestDeviceConfigOrHealthStatus	 -- A request for a device configuration or health status report for one or more devices at a site, or the comm connections of a device. Also used to request a device parameter setting change.

RequestScanData	-- A request for data from a radiation scan.

RequestContainerListSummary	-- A request for data regarding containers at a site.

RequestSiteConfigData	-- A request for data regarding a site.

TraversalSummaryRequest	-- A request for data regarding a traversal.

CBRNCaseRequest	-- A request for information about a CBRN case, or case data files.

ReachbackRequest	-- A request for reachback assistance in adjudicating a detection alarm.

Detailed notes on each of these messages are provided in later sections of this document.

## IEPD Artifacts

The N.25 IEPD artifacts for this message are:

- This document

- NIEM compliant XML exchange schemas (unconstrained and constrained)

- Component Mapping Template

- Visual model of the message schema

- Sample Messages

The detailed list of the digital data files that comprise the N.25 IEPD artifacts for this message is contained in the IEPD Catalog.

## Reference Documents

The following table lists the reference documents for this message.

Table 1.  Reference Documents

| Document Type | Date | Document Title |
| --- | --- | --- |
| Master Document |  | N25 IEPD Main Document-Volume 4-Radiation Device Message |
| Master Document |  | N25 IEPD Main Document-Volume 6-Site Operations Message |
| Master Document |  | N25 IEPD Main Document-Volume 8-Device Configuration and Health Status Message |

# Business Scenario

Each request message has a corresponding response that is defined in another exchange schema.  The description of each request message explains the corresponding response message and references the volume of this Master Document that provides more details on the response.

The RequestMessage schema comprises message which are requests to the data collection systems for information on several key data, settings and configuration parameters.  Examples Requests are (1) Threat level change on the devices (2) Global Configuration at a port (3) one-to-many container(s) summary (4) Health status (5) Scan data (6) ThreatLevel Status on a device.  The request messages could either be a result of a system event or a user query.

# Business Requirements

The basic business requirement for this message is to enable requests for various sets of information.  In the N.25 message protocol, requests messages have been generally grouped into one xml schema.  A common set of elements for a request message are defined for standard use in all request messages. Then each unique request message adds the specific elements needed.

# Business Processes and Rules

A business process model is not provided for all the request messages since there are many circumstances in which it may be determined that a request message is to be transmitted. For understanding of the general flow, the following pages provide a process description and diagram for one message, the SiteInfoRequest message. For completeness, the response message to this request is also described.

The following table and accompanying diagram provides a description of the process of requesting and receiving information about a detection site.

Table 2. Site Info Request-Response Process and Rules

| NBR | Activity | Description | Business Rules |
| --- | --- | --- | --- |
| 1 | Request Site Info | The process starts when the requesting user determines that site info data is needed and initiates the request via his/her U/I. The response message always provides site identity, location, and status. The data that the user may also request includes: - Site controller ID - Subsite information - Device configuration information | The following options will be provided to the requesting user to request site info data: 1. Selected Site – The user will be able to select one or more sites from a list of known sites. 2. Selected Area - The user will be able to select one or more areas from a list of areas, to request all sites in the area(s). 3. Selected Locale – The use will be able to input a lat/long coordinate and radius (km or miles), to request all sites within the locale that is defined by the input data. 4. All Sites – The user will be able to request info for all sites known to the user. 5. Discovery – The user will be able to request discovery of new sites (eg, sites not currently in the list of known sites). |
| 2 | Validate & log | The user’s input requesting a message be sent is validated and logged. Any errors will be fedback to the user via the U/I to correct. |  |
| 3 | Generate Request | The outbound request message is generated. |  |
| 4 | Send Message | The message is transmitted. |  |
| 5 | Update U/I | The user’s U/I is updated to reflect the transmission of the request message. |  |
| 6 | Receive Message | The detection site receives the message. |  |
| 7 | Validate and Log | The message is logged and validation rules are applied to it. The results of the validation checks are then used to determine subsequent processing. | Rules are needed for the following validationchecks. 1. The source of the message is validated as authentic and authorized to make the request. 2. The message is properly formatted, has no data errors, and has sufficient information to enable the request to be processed. 3. The message no unknown or otherwise unacceptable content that could represent a security threat to the system. If the message passes all checks, proceed to “Prepare Response”, otherwise proceed to “send error msg”. |
| 8 | Send Status Msg | Provide a response message to the originator of the success or failure to accept the message. | If the validation check failure is rule 1 or 3, then do not send a message to the originator. Instead, alert the designated ISSO. |
| 9 | Correct Message | The requesting system receives an error response and takes appropriate action to validate/log, correct the error, and resend. |  |
| 10 | Wait for reply | The requesting system awaits a response to the request |  |
| 11 | Advise Timeout | If the wait times out, the result will be logged and the user notified via the user’s U/I. | If the user wants to try again, a new message must be requested. The original message is terminated if a timout occurs. |
| 12 | Prepare Response | The detection site prepares a response to the request. |  |
| 13 | Send Response | The detection site sends the response message. |  |
| 14 | Receive Message | The requesting system receives the message. |  |
| 15 | Validate and Log | The message is logged and validation rules are applied to it. The results of the validation checks are then used to determine subsequent processing. | Rules are needed for the following validationchecks. 1. The source of the message is validated as authentic and authorized to make the request. 2. The message is properly formatted, has no data errors, and has sufficient information to enable the request to be processed. 3. The message no unknown or otherwise unacceptable content that could represent a security threat to the system. If the message passes all checks, then: - Update the user’s U/I with the data from the detection site’s response - Clear the Wait timer for this message - If more than one message was sent (multiple sites), and all waits have not been ended, then continue to wait, otherwise, the process ends. Otherwise proceed to “send error msg” and reset the Wait timer for this site’s response message. However, if the validate check failed on items 1 & 3 (security problems). In this case the ISSO is notified, and an error message is not sent to the site. |
| 16 | Clear Wait | Clear the Wait time for the message that has been successfully received. |  |
| 17 | Update U/I | Update the user’s U/I with the information received in the message. |  |
| 18 | Reset Wait | Reset the Wait timer for a message that has been determined to be a error. |  |
| 19 | Send Status Message | Send a status response to the message originator. | If the validation check failure is rule 1 or 3, then do not send an error status message to the originator. Instead, alert the designated ISSO. |
| 20 | Correct Message | The detection site corrects the message and resends. |  |

The following diagram illustrates these process activities, with the activity name and number in the above table repeated in the diagram.

Figure 1.  SiteInfo Request-Response Process

# Implementer Notes

## General

1. The schema outline provided in Appendix A provides a structured list of the desired schema elements and attributes.  Examination of the schema with an xml tool such as XML Spy or Stylus Studio will reveal more elements/attributes than appear in this outline. This is due to type inheritance that occurs with imports from referenced schema.  There is no requirement to implement any elements/attributes that do not appear in the schema outline in this document, unless specifically addressed in the detailed notes in this section.

2. The names in the schema outline in Appendix A are the xml tagged element names that will appear in a message instance. A name enclosed in parentheses following the element name is an attribute.

3. Indentation is used in the schema outline to show parent-child nesting.

4. The schema outline includes element cardinality rules on the same line with the element, enclosed in brackets. The values and meanings are:

[1,1]  The element is required and there may be only one instance.

[0,1] The element is optional and there may be zero or one instance.

[0 or many]  The element is optional and there may be zero, one or many instances.

In some cases if an optional element is used, then a child element may be required. The rules for these are specified in the details in this section.

5. The descriptions explaining each element and attribute is contained in the schema xsd file in a ‘xsd:documentation’ statement immediately following the declaration of the element, attribute or type.  The information in this section 5 is supplementary to the documentation in the schema xsd file.

6. Two schema files are provided for each message: unconstrained and constrained. They are denoted by the schema filename suffix ‘-x’ and ‘-xc’ for unconstrained and constrained respectively.  The difference between the two is that the constrained schema includes cardinality constraints and other types of constraints, such as max and min values, that may be applicable. The constrained schema is used for the messages defined in this N.25 IEPD.  The unconstrained schema is provided as part of the general NIEM concept of share and reuse of data definitions by other domains and IEPDs.

## Common

The following notes are common to all request messages.

<ActivityName> – This optional element, which is included in the RequestData element, provides a way to record the process or procedure from which the request originated.

<ConveyanceRegistrationPlateInfo> – See ConveyanceObservationReport Details above.

<UserName> - If this optional element is used, one of the alternatives for user name must be used.  If <PersonName> is used, <PersonSurName> must always be used.

Message Content Header:  All messages have a common header.  The following provide special notes on the elements of the header:

<MessageOrigin> -  To identify the message origin, at least one of <FacilityID>, <FacilityName>, or <OrganizationIdentification>, <OrganizationName> must be provided.

<MessageDestination> - At least one destination is required.A message may be sent to multiple destinations by including this element for each destination with at least one of  <FacilityID>, <FacilityName>, or <OrganizationIdentification>, <OrganizationName>.

Coordinated Universal Time (UTC) shall be used in all date and time elements.  XML standards use the dateTime format based on the ISO 8601 dateTime [YYYY-MM-DDTHH:MM:SSZ] where the “Z” is appended to indicate Universal Time (UTC). Note that in the event of times/durations involving fractions of a second, the separator between seconds and fractions of seconds may be a comma or the full stop, eg. “.”.  The number of fraction digits shall not exceed three (3).

The information in each of the following sections supplements the information in the schema for the identified elements.  If an element is not discussed, this means that there is no supplementing information.

## RequestDeviceConfigOrHealthStatus Details

### General

The elements of this message enable the user to generate a variety of requests regarding a device or collection of devices at a detection site.  The following subsections describe every combination of these elements and their meaning with respect to the response report that is being requested.

The following reports may be requested. The reports are defined in the DeviceConfigOrHealthStatusMessage.xsd (see Master Document Volume 8 for details).

- Device Assembly Connections Report

- Device Comm Connections Report

- Device Health Status Report

- Device Comm Health Status Report

- Device Configuration Parameters Report

The elements and their values to be used to request each of the device configuration and health status reports is described in the following subsections.

### Request a Device Assembly Connections Report

This request is for the assembly configuration of the device. This is all the other devices connected to the devices by a connection kind of "Subassembly" or "Component". Note that "device" is a generic term and includes site/subsite controllers/systems.

Table 3. Element Values to Request Device Assembly Connections Report

| Element Name | Value of the Element |
| --- | --- |
| DeviceAssociationKindCode | Use 'Subassembly" to get the devices first level of subassemblies. Use 'Component' to get the device's assembly breakdown to the lowest level components. |
| ConfigStatus KindCode | Configuration |
| RadEncounter DeviceID | Mandatory unless RequestScopeCode=All. In that case this element is omitted. |
| EncounterDeviceCategoryCode | Mandatory unless RequestScopeCode=All. In that case this element is optional, and would be used to restrict the scope of the devices desired. |
| DeviceParameter Name | Omitted |
| DeviceParameter Value | Omitted |
| unitsText | Omitted |

The following provides the DeviceAssociationKindCode table definitions.

Table 4. DeviceAssociationKindCode Table

| Code | Description |
| --- | --- |
| Subassembly | An assembly-subassembly pair |
| Component | An assembly/subassembly-component pair, where the child item in the pair is atomic, ie, cannot be further decomposed |
| Wired Comm | The devices are physically hardwire connected at the same location. |
| Wireless Comm | The devices are physically wirelessly connected at the same location. |
| Other Comm | The devices are at separate locations and communicate with each other, such as communicating over a WAN. |
| Comm | The devices have a communication link of unspecified kind. |
| Kit | The devices are not connected but are part of an operational configuration; both devices are needed to meet the mission requirement. |
| NA | Not Applicable |

### Request a Device Comm Connections Report

The request is for the communications connections of the device. This is all the other devices connected to the device by a kind of communications connection. Note that "device" is a generic term and includes site/subsite controllers/systems.

Table 5. Element Values to Request Device Comm Connections Report

| Element Name | Value of the Element |
| --- | --- |
| DeviceAssociationKindCode | Choices of the types of communications connections that may be requested are: Wired Comm Wireless Comm Other Comm Comm |
| ConfigStatus KindCode | Configuration |
| RadEncounter DeviceID | Mandatory unless RequestScopeCode=All. In that case this element is omitted. |
| EncounterDeviceCategoryCode | Mandatory unless RequestScopeCode=All. In that case this element is optional, and would be used to restrict the scope of the devices desired. |
| DeviceParameter Name | Omitted |
| DeviceParameter Value | Omitted |
| unitsText | Omitted |

### Request a Device Health Status Report

This request is for the health status of a device.

Table 6. Element Values to Request a Device Health Status Report

| Element Name | Value of the Element |
| --- | --- |
| DeviceAssociationKindCode | Subassembly |
| ConfigStatus KindCode | Status |
| RadEncounter DeviceID | Mandatory unless RequestScopeCode=All. In that case this element is omitted. |
| EncounterDeviceCategoryCode | Mandatory unless RequestScopeCode=All. In that case this element is optional, and would be used to restrict the scope of the devices desired. |
| DeviceParameter Name | Omitted |
| DeviceParameter Value | Omitted |
| unitsText | Omitted |

### Request a Device Comm Health Status Report

This request is for the health status of a device's communications connections.

Table 7. Element Values to Request a Device Comm Health Status Report

| Element Name | Value of the Element |
| --- | --- |
| DeviceAssociationKindCode | Choices of the types of communications connections for which status may be requested are: Wired Comm Wireless Comm Other Comm Comm |
| ConfigStatus KindCode | Status |
| RadEncounter DeviceID | Mandatory unless RequestScopeCode=All. In that case this element is omitted. |
| EncounterDeviceCategoryCode | Mandatory unless RequestScopeCode=All. In that case this element is optional, and would be used to restrict the scope of the devices desired. |
| DeviceParameter Name | Omitted |
| DeviceParameter Value | Omitted |
| unitsText | Omitted |

### Request a Device Config Parameter Report

This request is for the current value of a device's parameter(s). The desired parameter name(s) must be provided, unless RequestScopeCode = All.

Table 8. Element Values to Request a Device Configuration Parameter Report

| Element Name | Value of the Element |
| --- | --- |
| DeviceAssociationKindCode | Subassembly |
| ConfigStatus KindCode | Parameter |
| RadEncounter DeviceID | Mandatory unless RequestScopeCode=All. In that case this element is omitted. |
| EncounterDeviceCategoryCode | Mandatory unless RequestScopeCode=All. In that case this element is optional, and would be used to restrict the scope of the devices desired. |
| DeviceParameter Name | Mandatory for each parameter for which a parameter's value is requested. The string "ALL" shall be entered to indicate that all parameters are requested. |
| DeviceParameter Value | Omitted |
| unitsText | Omitted |

<DeviceParameterName> -- A choice must be made whether to use the DeviceParameterVendorName or the DeviceParameterName (a standard name that maps to vendors’ name). Note: the standard DeviceParameterName “ThreatLevelSetting” is reported/changed using this element. Note:  the standard parameter names are currently not contained in a codelist since the set of all parameters names has not been defined.

<DeviceParameterValue> -- Note that this is a substitution group where the element tag to use is determined by the base data type of the value. The following identifies which tag(s) to use based on the base data type of the parameter:

Table 9. DeviceParameterValue Choices

| Element | Useage |
| --- | --- |
| <DeviceParameterPointValue> | Used for decimal values, or integers and floats expressed as decimal values. |
| <DeviceParameterMaxValue> | Used to express the high end of a range of deciimal values, or integers and floats expressed as decimal values. |
| <DeviceParameterMinValue> | Used to express the low end of a range of deciimal values, or integers and floats expressed as decimal values. |
| <DeviceParameterValueIndicator> | Used for boolean values. |
| <DeviceParameterValueText> | Used for string values that are not enumerations from a codelist. |
| <DeviceParameterValueListText> | Used for string values that are enumerations from a code list. |
| <DeviceParameterValueListName> | Used to provide the name of the codelist in which an enumeration exists. |

The last two in the list must be used together.  For example, in the case of the “ThreatLevelSetting” parameter, the element tags would appear as:

<DeviceParameterValueListText>enumeration item</<DeviceParameterValueListText>

<DeviceParameterValueListName>ThreatLevelDeterminationCode</DeviceParameterValueListName>

### Request a Device Comm Connection Parameter Report

This request is for the current value of a parameter of a device's communication connection (comm path component). The desired parameter name(s) must be provided, unless RequestScopeCode = All.   Note: This is a future report not currently included in N.25.

Table 10. Element Values to Request a Device Comm Parameter Report

| Element Name | Value of the Element |
| --- | --- |
| DeviceAssociationKindCode | Choices of the types of communications connections for which status may be requested are: Wired Comm Wireless Comm Other Comm Comm |
| ConfigStatus KindCode | Parameter |
| RadEncounter DeviceID | Mandatory unless RequestScopeCode=All. In that case this element is omitted. |
| EncounterDeviceCategoryCode | Mandatory unless RequestScopeCode=All. In that case this element is optional, and would be used to restrict the scope of the devices desired. |
| DeviceParameter Name | Mandatory for each parameter for which a parameter's value is requested. The string "ALL" shall be entered to indicate that all parameters are requested. |
| DeviceParameter Value | Omitted |
| unitsText | Omitted |

## RequestScanData Details

This message is used to request radiation scan data from a detection site collected by scanning some object, such as a vehicle, container, or airplane.

<DetectionEventUUID> -  Unique identifier of the detection event(s) for which data is requested. If the data is available, this will result in a RadiationDeviceMessage (see Master Document Volume 4 for details), with the requested scan data. The <RequestScopeCode> element is used to indicate if the desired scan data is for the RadiationMeasurement or the RadiationAssessment.  If the desired response is for the radiation measurement data, use the scope code “ScanMeasurementDataRequest”.  If the desired response is for the radiation assessment data, use the scope code “ScanAssessmentDataRequest”.

## RequestContainerListSummary Details

If the data is available, this request will result in a Container Summary Report providing details of one or more containers currently present at the site (see Master Document Volume 6 - Site Operations Information Message for details).

<RequestScopeCode> -- This element is used to define what is desired in the response message.  The code values to use are explained in the following table:

Table 11. Container List Scope Codes

| Code Value | Useage |
| --- | --- |
| SingleRequest | The request is for data regarding a single container described by the ContainerID in the request. |
| PartialRequest | The request is for data regarding multiple containers (but less than all possible), as described by the ContainerIDs in the request. |
| FullRequest | The request is for data regarding all contain for which information is available. In this case, the ContainID element is not used in the request message. |

## RequestSiteConfigData Details

This message is used to request information about a detection site (for information about the devices at a site, use the RequestDeviceConfigOrHealthStatus message described above). The response to this request is the SiteInfoReport (see Master Document Volume 6 - Site Operations Information Message for details).

<RequestScopeCode> - This element is used to indicate the desired site info.  If set to “SingleRequest”, only information about the site itself is requested.  If set to “FullRequest”, then information about the site and all its subsites (if any) is requested.  The enumeration “PartialRequest” is not applicable.

## TraversalSummaryRequest Details

This message is used to request information about a traversal.

NOTE: A traversal is the end-to-end workflow of a conveyance through an encounter process. Zero, one or more detection events may occur during a traversal.

The response to this request is the TraversalSummaryReport (see Master Document Volume 6 - Site Operations Information Message for details). A report may be requested for one or more traversals from the requested site.

<RequestScopeCode> - This element is not applicable for this request.

## CBRNCaseRequest

The following definitions are provided to aid in understanding the concept of a Case.

| Case :: | An aggregation of information about a set of related activities and events. |
| --- | --- |
| Traversal :: | The end-to-end workflow of a conveyance through an encounter process. One or more detection events occur during a traversal. |
| Detection Event :: | The set of all data collected during an event that involves the inspection of an Item(s) for the purpose of detecting the presence of illicit goods and materials. This includes data collected by the device(s) used to perform the detection as well as information input by operator(s) involved in the detection activities that the event involves. A detection event can occur in many venues, such as a CBP Port of Entry, a USCG boarding, a state operated weigh station on a US highway, and a general aviation inspection at an airport in Mexico or Canada. |
| Encounter :: | The probabilistic occurrence that an illegal rad/nuc object or material will be put into a position through inspection activities where it can be discovered or scanned and detected by a sensor. |

A Case consists of data about the case and data about the objects of interest to the Case.

These are referred to in this IEPD as the Case Description and the Case Contents.  The contents may be collections of data contained in digital data files or other Cases.

This message provides for the following kinds of request for Case data:

Table 12. Case Request Kinds

| Kind of Request | Description |
| --- | --- |
| Files by ID | Request for one or more case files based on file id. |
| Files by Subject | Request for all files that are of a specific subject matter type (one or more). |
| Case Content | Request for the list of files and other cases recorded as part of a case. |
| Case Description | Request for the data that describes the case (and its metadata). |
| Associated Cases | Request for the cases associated with a case. |
| Case by Status | Request for all cases with a specific status code. |
| Case by Threat Level | Request for all cases with a specific threat level code or higher, for a specified case status code. |

The following table identifies the data elements that must be included for each request kind to completely define the requested case data.

Table 13. Case Request Required Data

| Kind of Request | Case UUID | Binary Subject Code | Binary File UUID | Case Threat LevelCode | Case Status |
| --- | --- | --- | --- | --- | --- |
| Files by ID | Optional | No | Yes | Optional | Optional |
| Files by Subject | Optional | Yes | No | Optional | Optional |
| Case Content | Yes | No | No | Optional | Optional |
| Case Description | Yes | No | No | Optional | Optional |
| Associated Cases | Yes | No | No | Optional | Optional |
| Case by Status | No | No | No | Optional | Yes |
| Case by Threat Level | No | No | No | Yes | Optional |

Yes = The property is mandatory.

No = The property is not required and should not be included in the request. If it is included it will be ignored.

Optional = The property is optional. If used, the response data will be restricted by the value of the property.

The following tables describe the Status and Threat Level codes:

Table 14. Case Status Codes

| Code | Description |
| --- | --- |
| Active-Threat | The case is active and determined to represent a threat. Requires total commitment of effort to monitor, track, and status. |
| Active-Hot | The case is active, but not yet determined to be a threat. Requires constant attention. |
| Active-Watch | The case is active of sufficient interest to maintain on a watch list. |
| Active-Routine | The case is active but not being proactively pursued. |
| Suspended | The case is no longer active, but can be returned to an active state by a time or event trigger, or user action. |
| Pending | The case has been created, but will not become active until triggered by a date-time, event, or user action. |
| Inactive | The case in no longer of interest, but a decision to close the case has not been made. |
| Merged | The case has been closed by merging into another case. |
| Closed-Normal | The case has been closed. |
| Cosed-Error | The case has been determined to have been opened in error and has been closed. |
| Unknown | The case status is unknown. |

Table 15. Case Threat Level Codes

| Code | Description |
| --- | --- |
| Severe Risk | Severe risk that the case represents activities or items intended to support a terrorist attack. |
| High Risk | High risk that the case represents activities or items intended to support a terrorist attack. |
| Significant Risk | Significant risk that the case represents activities or items intended to support a terrorist attack. |
| General Risk | General risk that the case represents activities or items intended to support a terrorist attack. |
| Low Risk | Low risk that the case represents activities or items intended to support a terrorist attack. |
| No Risk | The case represents activities or items that have been adjudicated by the governing authority to not represent a threat of terrorist attack. |
| Hazard | The case represents activities or items that have been adjudicated by governing authority to not represent a threat of terrorist attack, but do represent a risk to people and/or the environment. |
| Illicit | The case represents activities or items that have been adjudicated by governing authority to not represent a threat of terrorist attack, but are believed to be in violation of laws or regulations. |
| Unknown | The threat represented by the activities or items is unknown. |

The scope of a request is defined by the element: RequestScopeCode. The following is the meaning of the scope codes:

Table 16. Case RequestScopeCode

| Code | Description |
| --- | --- |
| SingleRequest | The request is for data regarding a single Case described in the request. |
| PartialRequest | The request is for data regarding multiple Cases (but less than all possible), as described in the request. |
| FullRequest | The request is for data regarding All Cases for which information is available, as described in the request. |
| NotApplicable | The nature of the request does not require a bounding scope for the response. |

## ReachbackRequest Details

This message is used by a detection site to request reachback assistance in adjudicating an alarm. There may not be a direct response to this message depending on how the reachback process is executed, which could take many forms depending on the detection site making the request, the nature of the alarm, the reachback support organization that is engaged, etc. Associated with this request is the ReachbackRecommendation message (see Master Document Volume 6 - Site Operations Information Message for details).

<RequestScopeCode> - This element is not applicable for this request.

<ReachbackRequestCode> - The following table provides an explanation of the element values:

Table 17. Reachback Request Code Useage

| Code | Description |
| --- | --- |
| ImageAnalysis | Initial results of image analysis appear abnormal or suspicious, therefore reachback assistance is requested. |
| RIID | Results from RIID scan appear abnormal or suspicious, therefore reachback assistance is requested. |
| ASP | Results from ASP scan appear abnormal or suspicious, therefore reachback assistance is requested. |
| GammaAlarm | An unresolved gamma alarm has occurred and reachback assistance is requested. |
| NeutronAlarm | An unresolved neutron alarm has occurred and reachback assistance is requested. |
| SuspiciousCargo | A cargo item has been identified as suspicious, therefore reachback assistance is requested. |
| Secondary | An item of interest has been directed to secondary inspection, and reachback assistance is requested. |

< ScanIdentificationData> -- If this optional element is used, then the following elements are required:

MeasuredItemID

MeasuredItemKindCode

TraversalID

DetectionEventUUID

# Schema Outline

RequestMessage [1,1]

MessageContentHeader [1,1]

MessageID [1,1]

MessageCreationDateTime [1,1]

MessageDispatchDateTime [1,1]

MessageKindCode [1,1]

MessagePriorityCode [0,1]

MessageOrigin [1,1]

Choice of two of the following

FacilityID [0,1]

FacilityName [0,1]

OrganizationIdentification [0,1]

OrganizationName [0,1]

MessageDestination [1,many]

Choice of two of the following

FacilityID [0,1]

FacilityName [0,1]

OrganizationIdentification [0,1]

OrganizationName [0,1]

MessageVersionText [0,1]

RemarkText [0,1]

MessageContent [1,1]

RequestDeviceConfigOrHealthStatus [0,1]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

ConfigStatusKindCode [0,1]

DeviceAssociationKindCode [0,1]

RequestedDevice [0,many]

EncounterDeviceID [0,1]

EncounterDeviceCategoryCode [0,1]

DeviceParameter (parameterUnitsText) [0,many]

DeviceParameterName [0,1]

DeviceParameterVendorName [0,1]

DeviceParameterSettableIndicator [0,1]

DeviceParameterStatusIndicator [0,1]

Choice of one or two of the following

DeviceParameterValueText [0,1]

DeviceParameterPointValue [0,1]

DeviceParameterMaxValue [0,1]

DeviceParameterMinValue [0,1]

DeviceParameterValueIndicator [0,1]

DeviceParameterValueListText [0,1]

DeviceParameterValueListName [0,1]

RequestScanData [0,1]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

DetectionEventUUID [0,many]

RequestContainerSummary [0,1]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

ContainerID [0,many]

MeasuredItemID [0,1]

IdAcquisitionMethodCode [0,1]

RequestSiteConfigData [0,1]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

FacilityID [0,1]

FacilityName [0,1]

RequestTraversalSummary [0,1]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

TraversalID [0,many]

CBRNCaseRequest [0,many]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

CaseUUID [0,1]

RadNucCaseRequestCode [1,1]

CaseRequestData [1,1]

BinaryFileUUID [0,many]

BinarySubjectCode [0,many]

CaseThreatLevelCode [0,many]

CaseStatusCode [0,many]

ReachbackRequest [0,many]

RequestID [1,1]

RequestDateTime [0,1]

RequestScopeCode [0,1]

ActivityName [0,1]

UserName [0,1]

Choice of one of the following

EmployeeIdentifier [0,1]

PersonName [0,1]

PersonNamePrefixText [0,1]

PersonGivenName [0,1]

PersonMiddleName [0,1]

PersonSurName [0,1]

PersonNameSuffixText [0,1]

PersonFullName [0,1]

ActivityUserLogonID [0,1]

RequestAgency [1,1]

RequestAgencyCode [0,1]

OrganizationName [0,1]

OrganizationIdentification [0,1]

RemarkText [0,1]

ReachbackRequestDateTime [0,1]

ReachbackRequestCode [0,1]

ScanIdentificationData [0,1]

RemarkText [0,1]

TraversalID [1,1]

DetectionEventUUID [1,1]

MeasuredItemID [1,1]

MeasuredItemIDKindCode [0,1]

MeasuredItemKindCode [1,1]

ReachbackReasonText [0,1]