Department of Homeland Security

Customs and Border Protection

And

Domestic Nuclear Detection Office

N.25 Radiation & Nuclear Detection Message Protocol

Information Exchange Package Documentation

Volume 4. Radiation Device Message

Document Number 500-IECBP-107350 Volume 4v1.1.40

14 February 2009

Change History

The following table indicates the changes that were made to this document since its last release. Note: version number is formed to correspond to the IEPD version number as follow:    MajorVersionNumber.MinorVersionNumber.BuildNumber.

| New Version | Modified By | Section | Page | Change Made |
| --- | --- | --- | --- | --- |
| 1.0.39 | W. R. Wright | All | All | Initial Release |
| 1.1.40 | CBP TASPO and W. R. Wright | All | All | Minor administrative updates |

Table of Contents

Change History	2

1	Overview	4

1.1	Purpose	4

1.2	Introduction	4

1.3	IEPD Artifacts	4

1.4	Reference Documents	5

2	Business Scenario	5

3	Business Requirements	6

4	Business Processes and Rules	6

5	Implementer Notes	6

5.1	Schema Outline	6

5.2	Common	7

5.3	Details	7

Appendix A.	Schema Outline	8

List of Tables

Table 1.  Reference Documents	5

N.25 Radiation Device Message

# Overview

## Purpose

This document provides details of the N.25 Radiation Device Message. The intent is to assist implementers understanding of the data requirements for this message type.

## Introduction

This message group provides an exchange for transmitting data collected during radiation detection events.  This includes data from radiation detection devices, other devices supporting the radiation detection process, and assessments of the nature of the threat (or absence thereof) posed by an item that triggers a radiation alarm(s).  There are three general groups of data that are included in this exchange schema.  All three may appear in a single message, or may be sent in separate messages.

- RadiationMeasurement- Provides data collected from radiation detection devices, supplemented by operator inputs.

- EventDataFiles- Provides data associated with a radiation measurement event that has been collected into files, typically from cameras and other imaging devices.

- RadiationAssessment- Provides data regarding threat assessment of a radiation measurement.

Detailed notes on each of these messages are provided in later sections of this document.

## IEPD Artifacts

The N.25 IEPD artifacts for this message are:

- This document

- NIEM compliant XML exchange schemas (unconstrained and constrained)

- Component Mapping Template

- Visual model of the message schema

- Sample Messages

The detailed list of the digital data files that comprise the N.25 IEPD artifacts for this message is contained in the IEPD Catalog.

## Reference Documents

The following table lists the reference documents for this message.

Table 1.  Reference Documents

| Document Type | Date | Document Title |
| --- | --- | --- |
| Master Document |  | Master Document Vol 2 – CBRN Domain |
| Specification |  | Radiation Data Format Specification for Homeland Security (600-DNDO-106900v0.11.doc) |
| Master Document |  | Master Document Vol 5 – General Device Message |

# Business Scenario

The typical scenario that involves this message is the occurrence of a detection event; i.e., the scanning of an item of interest, such as a shipping container, vehicle, airplane, railcar, or boat, for the presence of threat materials. The threat materials could be radioactive substances that could be fashioned into a “dirty bomb” or an Improvised Nuclear Device, or could be a nuclear weapon.  During the course of the encounter with the item of interest, radiation detection devices are normally employed, supported by other devices such as cameras.  The detection of radiation from the item of interest could trigger an alarm and require further examination and an assessment as to whether the alarm was caused by threat materials, normal commercial materials that pose no threat, or was a false alarm.  A very important consideration in the business scenario is the need to strictly minimize the impact on the normal flow commerce.

There are four general classes of data that are collected/produced during a detection event:

- Data collected by the radiation detection device(s)

- Data collected by Non-Intrusive Inspection (NII) devices, such as x-ray equipment.

- Data collected by supporting devices, such as cameras and bar code readers

- Data collected by human operators, such as CBP officers

- Assessments produced by officials on the scene, analysts performing evaluations of the data from various centers of expertise, and advanced devices that have a capability for probabilistic identification of radionuclides.

There are numerous detection event scenarios. The highest volume of such events occur at ports of entry/exit where the items of interest pass by fixed detectors, or mobile detectors (hand-held or mounted on some sort of vehicle) are moved along/over a stationary item of interest.

This Radiation Device Message is used to transmit data collected by radiological devices, supporting devices, operators, and assessments. Note: Data collected by NII devices is exchanged using the General Device Message (see Volume 5 of this series).  Some typical examples of radiation devices used in the CBP scanning scenarios are RPM (radiation portal monitors), RIID (RadioIsotope Identification Device), ASP (Advanced Spectroscopic Portal) and several other devices which can vary depending on the scanning context and location.

# Business Requirements

The basic business requirement for this message is to transmit detection event data from a detection site to other organizations for purposes of evaluation, assessment, and situational awareness.

# Business Processes and Rules

There are many processes that could result in the need to transmit this message, and are not documented herein.  The processes are determined by the organizations responsible for operating the detection sites (fixed and mobile).  Applicable rules may be found in the referenced document, Radiation Data Format Specification for Homeland Security (600-DNDO-106900v0.11.doc).

# Implementer Notes

## Schema Outline

1. The schema outline provided in Appendix A provides a structured list of the desired schema elements and attributes.  Examination of the schema xsd file with an xml tool such as XML Spy or Stylus Studio will reveal more elements/attributes than appear in this outline. This is due to type inheritance that occurs with imports from referenced NIEM schema.  There is no requirement to implement any elements/attributes that do not appear in the schema outline in this document, unless specifically addressed in the detailed notes in this section.

2. The names in the schema outline in Appendix A are the xml tagged element names that will appear in a message instance. A name enclosed in parentheses following the element name is an attribute.

3. Indentation is used in the schema outline to show parent-child nesting.

4. The schema outline includes element cardinality rules on the same line with the element, enclosed in brackets. The values and meanings are:

[1,1]  The element is required and there may be only one instance.

[0,1] The element is optional and there may be zero or one instance.

[0 or many]  The element is optional and there may be zero, one or many instances.

In some cases if an optional element is used, then a child element may be required. If a parent element is optional (ie, cardinality of [0,1 or more]) and a child element is indicated as mandatory (eg., [1,1]) this means that the child element is required only if the optional parent element is included in the message.  Rules more specific than this are described in the details in the sections.

5. The descriptions explaining each element and attribute is contained in the schema xsd file in a ‘xsd:documentation’ statement immediately following the declaration of the element, attribute or type.  The information in this section 5 is supplementary to the documentation in the schema xsd file.

6. Two schema files are provided for each message: unconstrained and constrained. They are denoted by the schema filename suffix ‘-x’ and ‘-xc’ for unconstrained and constrained respectively.  The difference between the two is that the constrained schema includes cardinality constraints and other types of constraints, such as max and min values, that may be applicable. The constrained schema is used for the messages defined in this N.25 IEPD.  The unconstrained schema is provided as part of the general NIEM concept of share and reuse of data definitions by other domains and IEPDs.

## Common

Coordinated Universal Time (UTC) shall be used in all date and time elements.  XML standards use the dateTime format based on the ISO 8601 dateTime [YYYY-MM-DDTHH:MM:SSZ] where the “Z” is appended to indicate Universal Time (UTC). Note that in the event of times/durations involving fractions of a second, the separator between seconds and fractions of seconds may be a comma or the full stop, eg. “.”.  The number of fraction digits shall not exceed three (3).

Message Content Header:  All messages have a common header.  The following provide special notes on the elements of the header:

<MessageOrigin> -  To identify the message origin, at least one of <FacilityID>, <FacilityName>, or <OrganizationIdentification>, <OrganizationName> must be provided. Normally this is the detection site name and/or identifier for this message.

<MessageDestination> - At least one destination is required.A message may be sent to multiple destinations by including this element for each destination with at least one of  <FacilityID>, <FacilityName>, or <OrganizationIdentification>, <OrganizationName>.

The information in each of the following sections supplements the information in the schema for the identified elements.  If an element is not discussed, this means that there is no supplementing information.

## Details

Specific details regarding this message are extensive, and are separately documented in the “Radiation Data Format Specification.” (See section 1.4, Reference Documents.) NOTE: this referenced document provides the baseline for a proposed revision to ANSI Standard N42.42, “Data format standard for radiation detectors used for Homeland Security.”  This document defines two data format standards: Radiation Measurement Data Format (RMDF) and Radiation Assessment Data Format (RADF).  These two formats are proper subsets of the Radiation Device Message described in this document.

Note:  Detection event data that is recorded in digital data files may be transmitted via the submessage defined by <EventDataFiles> in this message schema, or alternately via the CaseFileReport submessage of the Site Operations Info message (see Volume 6 of this Master Document series).  The message for transmitting files is determined by organizational and site-specific procedures.

# Schema Outline

RadiationDeviceMessage [1,1]

MessageContentHeader [1,1]

MessageID [1,1]

MessageCreationDateTime [1,1]

MessageDispatchDateTime [1,1]

MessageKindCode [1,1]

MessagePriorityCode [0,1]

MessageOrigin [1,1]

Choice of two of the following

FacilityID [0,1]

FacilityName [0,1]

OrganizationIdentification [0,1]

OrganizationName [0,1]

MessageDestination [1,many]

Choice of two of the following

FacilityID [0,1]

FacilityName [0,1]

OrganizationIdentification [0,1]

OrganizationName [0,1]

MessageVersionText [0,1]

RemarkText [0,1]

MessageContent [1,many]

RadiationMeasurementData [1,many]

DetectionEventData (dataFormatCode, detectionEventCategoryCode, detectionEventKindCode) [1,1]

ScanIdentifierData [1,1]

RemarkText [0,1]

TraversalID [1,1]

DetectionEventUUID [1,1]

MeasuredItemID [1,1]

MeasuredItemIDKindCode [0,1]

MeasuredItemKindCode [1,1]

EventOnsetDateTime [0,1]

DetectionEventLocation [0,1]

DetectionEventSiteID [0,1]

RadiationDeviceData (dataFormatCode) [0,1]

RemarkText [0,1]

DetectionEventDeviceInformation [1,1]

EncounterDeviceClassification [0,1]

EncounterDeviceDescriptionText [0,1]

EncounterDevice (radEncounterDeviceModeCode, radEncounterDeviceOperatingCode) [1,1]

RemarkText [0,1]

EncounterDeviceCategoryCode [0,1]

EncounterDeviceCategoryLevelCode [0,1]

Choice of one of the following

EncounterDeviceID [0,1]

EncounterDeviceMfgID [0,1]

EncounterDeviceAssetID [0,1]

ItemMakeName [0,1]

ItemModelName [1,1]

EncounterDeviceVersion [0,1]

VersionKindCode [0,1]

CurrentVersionID [0,1]

PriorVersionID [0,1]

EncounterDeviceStatus [0,1]

EncounterDeviceStatusDateTime [1,1]

OperationalIndicator [1,1]

EncounterDeviceHealthStatusCode [0,1]

EncounterDeviceStatusParameters [0,many]

ParameterNames [1,1]

ParameterValues [1,1]

EncounterDeviceConfiguration [0,1]

DeviceParameter (parameterUnitsText) [0,many]

DeviceParameterName [0,1]

DeviceParameterVendorName [0,1]

DeviceParameterSettableIndicator [0,1]

DeviceParameterStatusIndicator [0,1]

Choice of two of the following

DeviceParameterValueText [0,1]

DeviceParameterPointValue [0,1]

DeviceParameterMaxValue [0,1]

DeviceParameterMinValue [0,1]

DeviceParameterValueIndicator [0,1]

DeviceParameterValueListText [0,1]

DeviceParameterValueListName [0,1]

EncounterDeviceProbe [0,1]

EncounterDeviceProbeKindCode [0,1]

EncounterDeviceProbeModelName [0,1]

EncounterDeviceProbeID [0,1]

EncounterDeviceQualityControl [1,1]

RemarkText [0,1]

InspectionDate [1,1]

InServiceIndicator [1,1]

DetectionEventDeviceMeasurementData [1,many]

RemarkText [0,1]

RadEncounterDeviceMeasurementID [1,1]

RadEncounterDeviceLocation [1,1]

RemarkText [0,1]

RadEncounterDeviceLocationName [0,1]

RadEncounterDeviceCoordinates (dataQualityCode) [0,many]

CoordinateDateTime [0,1]

CoordinateDuration [0,1]

GeographicCoordinates (cepValue) [0,1]

Latitude [1,1]

LatitudeDegreeValue [1,1]

LatitudeMinuteValue [0,1]

LatitudeSecondValue [0,1]

Longitude [1,1]

LongitudeDegreeValue [1,1]

LongitudeMinuteValue [0,1]

LongitudeSecondValue [0,1]

Elevation [0,1]

Choice of one of the following

MeasurePointValue [0,1]

MeasureRangeValue [0,1]

MeasureEstimatedIndicator [0,1]

LengthUnitCode [0,1]

GeographicDatumCode [0,1]

CoordinatesMethodCode [0,1]

RadEncounterDeviceToItemDistance [0,1]

Choice of one of the following

MeasurePointValue [0,1]

MeasureRangeValue [0,1]

MeasureEstimatedIndicator [0,1]

LengthUnitCode [0,1]

RadEncounterDeviceToItemBearing (dataQualityCode) [0,1]

AzimuthOffset [1,1]

ElevationOffset [1,1]

RadEncounterDeviceSpatialOrientation (dataQualityCode) [0,1]

TrueNorthOffset [0,1]

VerticalOffset [0,1]

RotationalOffset [0,1]

MeasuredItemID [0,1]

PositionID [0,1]

RadEncounterDeviceRawData (occupiedIndicator) [1,1]

RemarkText [0,1]

MeasurementStartDateTime [1,1]

MeasurementStopDateTime [1,1]

RealTimeDuration [1,1]

ItemSpeed [0,1]

Choice of one of the following

MeasurePointValue [0,1]

MeasureRangeValue [0,1]

MeasureEstimatedIndicator [0,1]

RemarkText [0,1]

VelocityUnitCode [0,1]

ItemPosition [0,1]

Choice of one of the following

MeasurePointValue [0,1]

MeasureRangeValue [0,1]

MeasureEstimatedIndicator [0,1]

LengthUnitCode [0,1]

SpectrumData (sampleNumber, spectrumKindCode, calibrationMeasurementUUID, dataQualityCode) [0,many]

RemarkText [0,1]

Choice of one of the following

ItemSpectrumID [0,1]

CalibrationSpectrumID [0,1]

BackgroundSpectrumID [0,1]

RefBackgroundSpectrumID [0,many]

RefCalibrationSpectrumID [0,1]

DetectorDesignatorText [1,1]

DetectorKindText [1,1]

LiveTimeDuration [1,1]

DwellDuration [0,1]

ElapsedSweepsQuantity [0,1]

RadSourceKindCode [1,1]

SpectrumChannelData (compressionCode, listModeCode, channelQuantity) [1,1]

CountValues [1,1]

EnergyCalibrationAdjustCode [0,1]

NonLinearityCorrection [0,1]

EnergyDeviationValues [1,1]

SpectrumOverflowValue [1,1]

GrossCountData (dataQualityCode, endEnergyValue, startEnergyValue, radEnergyUnitsCode) [1,many]

RemarkText [0,1]

DetectorDesignatorText [1,1]

DetectorKindText [1,1]

LiveTimeDuration [1,1]

GrossCountQuantity [1,1]

BackgroundCountValues [1,1]

RefItemSpectrumID [0,1]

MaximumSigmaValue [0,1]

CountDoseData (calibrationMeasurementUUID, dataQualityCode) [0,many]

RemarkText [0,1]

DetectorDesignatorText [0,1]

DetectorKindText [0,1]

LiveTimeDuration [0,1]

RadCountRateValues (radCountRateUnitsCode) [0,1]

RadCountValues [0,1]

RadDoseRateValues (radDoseRateUnitsCode) [0,1]

RadTotalDoseValues (radDoseUnitsCode) [0,1]

RadAbsorbedDoseRateValues (radAbsorbedDoseRateUnitsCode) [0,1]

RadTotalAbsorbedDoseValues (radAbsorbedDoseUnitsCode) [0,1]

RadExposureRateValues (radExposureRateUnitsCode) [0,1]

RadTotalExposureValues (radExposureUnitsCode) [0,1]

RefCalibrationSpectrumID [0,many]

RadEncounterDeviceProcessedData [1,many]

RemarkText [0,1]

RawDataProcessingMethodDescriptionText [0,1]

RawDataStartStopIDList [1,1]

ProcessedSpectrumImage [0,1]

RemarkText [0,1]

BinaryCompressionDescriptionText [0,1]

BinaryObject [0,1]

Choice of one of the following

BinaryUTF8Object [0,1]

BinaryHexObject [0,1]

BinaryBase64Object [0,1]

ProcessedSpectrumData (endEnergyValue, radEnergyUnitsCode, startEnergyValue) [1,1]

RemarkText [0,1]

ProcessedSpectrumID [1,1]

ProcessedSpectrumDescriptionText [0,1]

DetectorDesignatorText [1,1]

DetectorKindText [1,1]

RealTimeDuration [1,1]

LiveTimeDuration [1,1]

ProcessedChannelData (compressionCode, listModeCode, channelQuantity) [1,1]

CountValues [1,1]

EnergyCalibrationAdjustIndicator [1,1]

NonLinearityCorrectionIndicator [1,1]

BackgroundSubtractionIndicator [1,1]

ProcessedGrossCountData (endEnergyValue, startEnergyValue, radEnergyUnitsCode) [0,1]

RemarkText [0,1]

ProcessedGrossCountDataDescriptionText [0,1]

DetectorDesignatorText [0,1]

DetectorKindText [0,1]

RealTimeDuration [0,1]

LiveTimeDuration [0,1]

NetCountRateValues [0,1]

BackgroundCountValues [0,1]

MaximumSigmaValue [0,1]

MaximumProcessedDataValue [0,1]

DetectionEventDeviceCalibrationData (radAbsorbedDoseUnitsCode, calibrationMeasurementUUID, calibrationKindCode, radCountRateUnitsCode, radDoseUnitsCode, radEnergyUnitsCode, radExposureUnitsCode, radFWHMUnitsCode, radDoseRateUnitsCode, radExposureRateUnitsCode, radAbsorbedDoseRateUnitsCode) [0,1]

RemarkText [0,1]

CalibrationID [0,1]

DetectorDesignatorText [0,1]

DetectorKindText [0,1]

CalibrationCreationDateTime [1,1]

CalibrationNuclide [0,many]

NuclideCode [0,1]

NuclideSourceID [0,1]

Equation [1,1]

RemarkText [0,1]

EquationCoefficients (subEquationNumber) [1,1]

EquationCovarianceMatrix [0,1]

EquationKindCode [1,1]

EquationDescriptionText [1,1]

UpperLimitValue [0,1]

LowerLimitValue [0,1]

ArrayXY [0,1]

RemarkText [0,1]

PointXY [0,many]

RemarkText [0,1]

XValue [1,1]

YValue [1,1]

XDescriptionText [0,1]

YDescriptionText [0,1]

VoltageAppliedValue [0,1]

UserEntryData [0,1]

RemarkText [0,1]

DetectionEventUUID [1,1]

MeasuredItem [0,1]

RemarkText [0,1]

CargoDescriptionText [0,1]

MeasuredItemIdentification [0,1]

RemarkText [0,1]

MeasuredItemID [0,1]

MeasuredItemIDKindCode [0,1]

IdAcquisitionMethodCode [0,1]

IdConfidenceCode [0,1]

EntryPersonID [0,1]

MeasuredItemKindCode [0,1]

MeasuredItemDescriptionText [0,1]

MeasuredItemQuantity (toleranceNumeric, quantityUnitText) [0,1]

MeasuredItemGeometryDescriptionText [0,1]

RadSourceActivity (nuclideActivityUnitsCode) [0,1]

MeasuredItemReferenceDate [0,1]

EncounterDeviceOperatorText [0,1]

EncounterDeviceOperatorComments [0,1]

DeviceOperatorID [1,1]

DeviceOperatorComments [1,1]

Shielding [0,1]

RemarkText [0,1]

MeasuredItemID [1,1]

LayerQuantity [0,1]

Layer [1,many]

LayerSequenceNumber [1,1]

ItemWidthMeasure [0,1]

Choice of one of the following

MeasurePointValue [0,1]

MeasureRangeValue [0,1]

MeasureEstimatedIndicator [0,1]

LengthUnitCode [0,1]

LayerDensityValue [0,1]

LayerMaterialDescriptionText [0,1]

LayerMaterialKindCode [0,1]

RadEncounterDeviceToShieldingDistance [0,1]

Choice of one of the following

MeasurePointValue [0,1]

MeasureRangeValue [0,1]

MeasureEstimatedIndicator [0,1]

LengthUnitCode [0,1]

EventDataFiles [0,many]

DetectionEventUUID [1,1]

EventDataFile [0,many]

BinaryDescriptionText [0,1]

BinaryLocationURI [0,1]

BinarySizeValue [0,1]

Binary [0,1]

RemarkText [0,1]

DetectionEventUUID [1,1]

BinaryFileUUID [1,1]

BinaryFileName [1,1]

BinaryCaptureData [0,1]

BinaryCaptureStartTime [0,1]

BinaryCaptureDuration [0,1]

BinaryCaptureDeviceID [0,1]

BinaryCaptureDeviceCategoryCode [0,1]

MIMEEncodingCode [0,1]

MIMEContentCode [0,1]

BinaryCompressionDescriptionText [0,1]

BinarySequence [0,many]

BinarySequenceIdentifier [1,1]

BinarySequenceDescriptionText [0,1]

ImagePerspectiveCode [0,1]

BinarySubjectCode [0,many]

Choice of one of the following

BinaryUTF8Object [0,1]

BinaryHexObject [0,1]

BinaryBase64Object [0,1]

BinaryMetadata [0,1]

DistributionText [0,many]

VersionIdentifier [0,1]

VersionEffectiveDateTime [0,1]

SecurityClassificationCode [0,1]

DocumentPrivacyActIndicator [0,1]

ClassificationReasonText [0,1]

ClassifiedByText [0,1]

DeclassDate [0,1]

DeclassEventText [0,1]

DeclassExceptionText [0,1]

DeclassManualReviewIndicator [0,1]

RadiationAssessmentData [0,many]

DetectionEventData (dataFormatCode, detectionEventCategoryCode, detectionEventKindCode) [1,1]

ScanIdentifierData [1,1]

RemarkText [0,1]

TraversalID [1,1]

DetectionEventUUID [1,1]

MeasuredItemID [1,1]

MeasuredItemIDKindCode [0,1]

MeasuredItemKindCode [1,1]

EventOnsetDateTime [0,1]

DetectionEventLocation [0,1]

DetectionEventSiteID [0,1]

RadiationAssessment [0,many]

DetectionEventAssessmentID [1,1]

DetectionEventAlarmData (convergenceDuration) [0,many]

RemarkText [0,1]

RadAlarmID [0,1]

RadAlarmDateTime [0,1]

RadAlarmLightCode [0,1]

RadAlarmAudibleIndicator [0,1]

AlarmDescriptionCode [0,1]

RadAlarmKindCode [0,1]

RadAlarmDuration [0,1]

RadAlarmSignalStrength [0,1]

DetectorDesignatorText [0,1]

DetectionEventAnalyzedMeasurementData [0,1]

RemarkText [0,1]

ProcessedSpectrumID [1,1]

ProcessedSpectrumImage [0,1]

RemarkText [0,1]

BinaryCompressionDescriptionText [0,1]

BinaryObject [0,1]

Choice of one of the following

BinaryUTF8Object [0,1]

BinaryHexObject [0,1]

BinaryBase64Object [0,1]

DetectionEventAnalysisResult [0,many]

RemarkText [0,1]

RefRadAlarmID [0,1]

ThreatDescriptionText [0,1]

ThreatLevelDeterminationCode [0,1]

AnalysisStartDateTime [0,1]

AnalysisEndDateTime [0,1]

AnalysisOrgCode [0,1]

OrganizationName [0,1]

AnalysisAlgorithm (parameterSetID) [1,1]

AlgorithmName [1,1]

AlgorithmVersion (algorithmTechnicalVersionText) [0,3]

VersionKindCode [0,1]

AlgorithmImplementationVersionID [1,1]

AlgorithmVersionReleaseDate [1,1]

AlgorithmVersionDescriptionText [0,1]

AlgorithmDescriptionText [0,1]

AlgorithmCreatorName [1,1]

NuclideIdentification [0,many]

Nuclide (radEnergyUnitsCode, nuclideBetaEndPointEnergyValue) [1,1]

NuclideCode [1,1]

NuclideCategoryCode [1,1]

NuclideActivityValue (nuclideActivityUnitsCode) [0,1]

NuclideIdConfidence [0,1]

Choice of one of the following

ConfidencePointValue (pointValueMargin) [0,1]

ConfidenceRangeValue [0,1]

RangeMaximumValue [0,1]

RangeMinimumValue [0,1]

ConfidenceCode [0,1]

NuclideEstimatedPosition [0,1]

RemarkText [0,1]

LengthUnitCode [0,1]

RelativePositionOriginText [0,1]

OffsetXValue [0,1]

OffsetYValue [0,1]

OffsetZValue [0,1]

NuclideEstimatedShielding [0,1]

AtomicNumberValue [0,1]

ArealDensityValue [0,1]

PhotopeakData (radEnergyUnitsCode, radFWHMUnitsCode) [0,1]

PhotopeakLibraryName [0,1]

PhotopeakEnergyValue [0,1]

PhotopeakNuclide [0,1]

NuclideCode [1,1]

PhotopeakIntensityValue [0,1]

PhotopeakNetAreaValue [0,1]

PhotopeakResolutionValue [0,1]

PhotopeakContinuumValue [0,1]

PhotopeakSigmaValue [0,1]

TemplateRankingData [0,1]

TemplateLibraryName [0,1]

TemplateRankedNuclide [0,many]

NuclideCode [1,1]

NuclideIdConfidence [1,1]

Choice of one of the following

ConfidencePointValue (pointValueMargin) [0,1]

ConfidenceRangeValue [0,1]

RangeMaximumValue [0,1]

RangeMinimumValue [0,1]

ConfidenceCode [0,1]

AlgorithmParameterSet (parameterSetID) [0,many]

RemarkText [0,1]

Choice of one of the following

ParameterSetList [0,1]

ParameterNames [1,1]

ParameterValues [1,1]

ParameterSetPair [1,many]

ParameterName [1,1]

ParameterValueText [1,1]

ParameterValueUnitsText [0,1]