Department of Homeland Security

Customs and Border Protection

And

Domestic Nuclear Detection Office

N.25 Radiation & Nuclear Detection Message Protocol

Information Exchange Package Documentation

Volume 1. Overview

Document Number 500-IECBP-107350 Volume 1v1.1.40

14 February 2009

Change History

The following table indicates the changes that were made to this document since its last release. Note: version number is formed to correspond to the IEPD version number as follow:    MajorVersionNumber.MinorVersionNumber.BuildNumber.

| New Version | Modified By | Section | Page | Change Made |
| --- | --- | --- | --- | --- |
| 1.0.39 | W. R. Wright | All | All | Initial Release |
| 1.1.40 | CBP TASPO & W. R, Wright | All | All | Change of name of RadNuc Domain to CBRN Domain. Added information on governance and stewardship of N.25 and the CBRN Domain. Change of name of Radiation Alarm Summary message to Alarm Summary message. |

Table of Contents

Change History	2

1	Executive Summary	5

2	N.25 IEPD Overview	5

2.1	Purpose	5

2.2	N.25 Background	5

2.3	CBRN Domain	6

2.4	ANSI N42.42 Background	6

2.5	NIEM Background	7

2.6	N.25 Relationship to other Messaging Standards	8

2.7	Versions	10

2.8	IEPD Artifacts	10

2.9	IEPD Metadata	10

2.10	Master Document Structure	11

3	Reference Documentation	12

4	N.25 Exchange	12

4.1	N.25 Purpose	12

4.2	Use Cases	12

4.3	Structural Organization of N.25 XML Schemas	12

4.4	NIEM Schemas	14

4.5	N.25 Extension Schema	14

4.6	N.25 Message Exchange Schemas	15

4.7	N.25 Exchange Constraint Schemas	16

5	Business Requirements	16

6	Business Rules	16

7	Process Flows and Business Processes	16

8	Business Scenarios	16

9	Domain Model	18

10	Testing and Conformance	18

11	Tools and Methodologies	19

11.1	IEPD Definition	19

11.2	Development Process Description	20

11.3	N.25 IEPD Approach	20

Appendix A.	IEPD Metadata	24

Appendix B.	IEPD Catalog	26

List of Figures

Figure 1.  N.25 / CBRN Domain S&L Communications Context	9

Figure 2. N.25 XML Schema Relationships	13

Figure 3. IEPD Development Process	20

Figure 4. Model Driven IEPD Development Approach	21

List of Tables

Table 1. Mapping of N.25 Messages to External Standards	8

Table 2.  Master Document Volumes	11

Table 3.  Reference Documents	12

Table 4. NIEM Schemas Used by the N.25 IEPD	14

Table 5. Extension Schema Used by the N.25 IEPD	15

Table 6. N.25 Exchange Schemas	15

Table 7. Count of Types, Elements, and Attributes	22

Table 8. N.25 IEPD Artifacts	23

# Executive Summary

N.25 is an electronic messaging protocol for exchange of data associated with radiation and nuclear detection activities.  The original version of N.25 was developed to support the Security Freight Initiative (SFI), which involves detection activities in foreign seaports to prevent threat materials from being included in cargo containers bound for the US.  The success of N.25 for SFI led to the decision to extend the protocol as a standard for all Customs and Border Protection (CBP) data exchanges involving radiation and nuclear detection activities.  Concurrently, the Domestic Nuclear Detection Office (DNDO) was defining data exchange messages for radiation and nuclear detection activities involving State, Local, and Tribal authorities (S&L) and other organizations, and the Department of Homeland Security (DHS) had established NIEM (National Information Exchange Model) as the standard information exchange framework for all DHS Components.  As a result, CBP and DNDO agreed to consolidate their messaging requirements into a NIEM-conformation version of N.25.

The current N.25 standard and subsequent versions and updates are being maintained by CBP/OIT/TASPO (Targeting and Analysis Systems Program Office) under the leadership of the Executive Director, CBP TASPO.  The N.25 IEPD incorporates the CBRN Domain as its extension schema.  DNDO/CIO is the steward of this domain for DHS.

In the NIEM terminology, the set of all artifacts that define and document the message protocol is referred to as an Information Exchange Documentation Package (IEPD). This document provides an overview of this new version of N.25.  This IEPD master document consists of Volume 1 addressing N.25 in general, with added volumes for the more detailed aspects of each message group.

# N.25 IEPD Overview

## Purpose

This document provides a general overview of the N.25 Message Protocol and the constituent artifacts of its defining IEPD.

## N.25 Background

N.25 is a messaging protocol designed and implemented by Customs and Border Protection for radiation detection operations in support of the Secure Freight Initiative.  As a result of the success of this initiative, N.25 is being extended for use at all CBP and other facilities that perform radiation detection activities.  As part of this extension, N.25 is being converted to be conformant with the NIEM XML standard.  Current version of the N.25 standard and subsequent version updates are published by CBP TASPO.

CBP is ultimately a major customer of the new radiation detection devices being developed by DNDO, and DNDO has been working to define messaging requirements for State, Local, and Tribal authorities and other organizatons. Therefore a joint DNDO-CBP effort was established to:

- Create a NIEM-conformant “RadNuc” domain addressing the data definitional needs of the global radiation detection mission (Note: this domain has subsequently been redesignated as the CBRN Domain (Chem, Bio, Rad, Nuc)).

- Convert the N.25 protocol based on this new XML domain model

- Harmonize N.25 and DNDO data format/messaging specifications

## CBRN Domain

The CBRN Domain is a NIEM-conformant schema intended to be the baseline definition of business objects required to support data interchange needs of the CBRN detection and interdiction mission area. Currently it is focused on the radiation/nuclear aspect of this mission.  Its initial definition has been developed by analysis of existing data specifications:  ANSI N42.42, ASP ICD1 and 2, HPRDS ICD1 and 2, and N.25.  The data types and elements defined in these specifications have been integrated and harmonized, and converted for NIEM conformance.  Specific N.25 message formats, such as the RadiationDevice message, are essentially tailored subsets of the CBRN Domain schema.

Additionally, the CBRN Domain will be proposed as the basis for a revised version of ANSI N42.42 that extends ANSI N42.42 to a broader set of radiation detection applications,  reduces the complexity and ambiguity of the existing data structures, and supports conformance with NIEM.

The CBRN Domain addresses a portion of the broader Chemical, Biological, Radiological, and Nuclear community.  As a result it is anticipated that the domain will be extended to address the exchange needs of the ChemBio protection mission as well as RadNuc incident management and response.  This could also result in an expansion of the N.25 message set to support the broader CBRN community.

The CBRN Domain provides the NIEM Extension Schema for the N.25 IEPD, and is therefore documented in Volume 2 of this IEPD Master Document.  The CBRN domain is planned for incorporation into the next release of NIEM as a mission domain.

## ANSI N42.42 Background

ANSI N42.42 is a first of its kind standard to identify a common set of data structures for sensor data from radiation detectors of many types.  This standard provides an excellent first approximation of radiation sensor data including gross counts, dose data and spectroscopic data.  However, the implementation of ANSI N42.42 was focused on fixed portal detection use cases and does not adequately address mobile radiation sensors.  Further, the data structures within ANSI N42.42 were flexible enough that valid implementations have been, more often than not, incompatible.  The CBRN Domain is intended to provide a revised set of data structures that extend the structures of ANSI N42.42 to a broader set of radiation detection missions while also simplifying the structure of the data and reducing unnecessary ambiguity.

## NIEM Background

DOJ and DHS launched NIEM on February 28, 2005. It leverages the data exchange standards efforts successfully implemented by DOJ’s Global Justice Information Sharing Initiative (Global) and extends the Global Justice XML Data Model (Global JXDM) to facilitate timely, secure information sharing across the whole of the justice, public safety, emergency and disaster management, intelligence, and the homeland security enterprise.

NIEM complies with the Homeland Security Presidential Directive (HSPD5), which assigns the Secretary of DHS the role of principal federal official for domestic incident management. The Homeland Security Act of 2002 charges the Secretary with the responsibility for coordinating federal operations within the United States to prepare for, respond to, and recover from terrorist attacks, major disasters, and other emergencies. In 2004, Congress passed the Intelligence Reform and Terrorism Prevention Act (IRTPA) and in 2005 Executive Order 13388 was issued by the President. Both items direct U.S. government organizations to strengthen the sharing of terrorism information between organizations and appropriate authorities of local and state governments and the protection of the ability of organizations to acquire this additional information.

The primary NIEM objectives are to:

- Bring stakeholders together to identify information sharing requirements for operational and emergency situations.

- Maintain a national model containing universal, common, and domain specific data components that pertain to agency information needs in order to facilitate development of an IEPD (Information Exchange Package Documentation – essentially an XML schema that defines a data format for a message/file.

- Develop standards, a common vocabulary, and an online repository of IEPDs to support information sharing.

- Provide technical tools to support development, discovery, dissemination, and reuse of IEPDs.

- Provide training, technical assistance, and implementation support services, as appropriate.

NIEM specifies a set of reusable components rendered in XML schemas as type, element and attribute definitions that comply with the W3C XML Schema specification. The resulting reference schemas provide a common language for describing data precisely. The constructs it defines are basic metadata building blocks – baseline data types and structural components. Users employ these building blocks to describe their own domain-oriented data semantics and structures. Rules that profile allowable XML Schema constructs and describe how to use them help ensure that those components are consistent and reusable.

The naming conventions and XML component structures used in this revision to the N.25 message set have been designed for comformance to the NIEM standards.

## N.25 Relationship to other Messaging Standards

The N.25 IEPD provides a standard set of information exchange messages that is conformant to the NIEM XML standard.  The IEPD defines message payloads and is independent of the technologies used to communicate the message payload. There are other messaging standards that define payloads that will also be used by the CBRN (Chemical, Biological, Radiological, and Nuclear) community.  The implementer’s of N.25 IEPD message types need to be aware of these related standards and avoid duplicating message types provided by these other standards that are already in use.  Principal among the related standards are IEEE 1512, CAP (Common Alerting Protocol), EDXL (Emergency Data Exchange Language), and UCORE (Universal Core). The following table identifies the related message types between N.25 and these standards, and describes the recommended approach for determining the appropriate message to be used.

Table 1. Mapping of N.25 Messages to External Standards

| External Standard | N.25 Message | Usage |
| --- | --- | --- |
| Common Alert Protocol (CAP) | Radiation Alarm Summary | When a radiation alarm summary represents an alert such that, by policy, an emergency operations center should be notified, the alarm may be translated into CAP to enable the EOC to receive it using existing capabilities. CAP should only be used if the alarm represents a valid alert, and, in that case, should be used in preference to a radiation alarm summary message. |
| IEEE 1512 | Radiation Alarm Summary | When a radiation alarm summary represents an alert such that, by policy, a dispatch center or transportation center should be notified, the alarm may be translated into an IEEE 1512 Hazmat message to enable the center to receive it using existing capabilities. |
| EDXL-RM | Device Configuration and Health | For centers that use EDXL-RM to track resources, a radiation device’s status and location may be provided via EDXL-RM. |
| EDXL-SA, Universal Core (UCORE), Cursor on Target (CoT) | Radiation Alarm Summary | These standards provide for very small messages with situational awareness information to be provided. Based on policy, the core elements from the Radiation Alarm Summary Message may be transmitted using any of these standards to stakeholders that support these formats. |

To support the conversion of N.25 exchanges as noted above,  XML translators (e.g.,  XSLT) are expected to be developed and supported.

The following figure illustrates the exchange of RadNuc payload data that is initiated as N.25 messages and is translated to other standard message types during the exchange process.  The figure demonstrates the communications architecture typical of U.S. state and local (S&L) radiation sensor deployments.  In this architecture, the deployment site injects a N.25 message into an XML router network compliant with the Emergency Data Exchange Language Distribution Element (EDXL-DE).  That network applies its distribution and business rules to the messages so that they reach various ‘edge node’ routers.   At the edge nodes, the messages may be converted using adaptors to communicate with Emergency Operations Centers, Dispatch Centers, and others using their own, native, preferred formats.

Figure 1.  N.25 / CBRN Domain S&L Communications Context

## Versions

Modifications to the N.25 message protocol may result in subsequent versions of this document. Each N.25 message group has been documented in its own volume of this master document, thus enabling version management at a lower level than the total IEPD should this be necessary from time to time. All versions of the documention will be published and made available depending upon security restrictions.  The Department of Homeland Security (DHS) Enterprise Data Management Office (EDMO) will maintain an accessible copy of the most current version of the documentation.

## IEPD Artifacts

The N.25 IEPD is a set of digital data files contained in a standard directory structure.  The files comprise two types of artifacts:

- Business, reference and supporting documentation

- Reusable technical documentation, principally XML schemas

The detailed list of N.25 IEPD artifacts is contained in the IEPD Catalog in Appendix B of this document.

## IEPD Metadata

IEPD Metadata is a NIEM standard set of information about the IEPD. The N.25 IEPD Metadata is contained in Appendix A of this document.

## Master Document Structure

This N.25 IEPD Master Document comprises the following volumes:

Table 2.  Master Document Volumes

| Volume | Title | Description |
| --- | --- | --- |
| 1 | IEPD Overview | Provides an overview and introduction to the N.25 IEPD. |
| 2 | CBRN Domain | Provides definition of standard data types and elements for the Chemical, Biological, Radiation, and Nuclear Detection mission areas, for interoperability and use in constructing information exchange messages. |
| 3 | Alarm Summary Message | Describes a message for transmitting alarm messages from a detection site. |
| 4 | Radiation Device Message | Describes a set of messages for transmitting data about a detection event and radiation data collected during the detection event. |
| 5 | General Device Message | Describes a message for transmitting data from device types other than radiation detectors collected during a detection event. |
| 6 | Site Operations Information Message | Describes a set of messages for exchanging data regarding operational activities at a detection site. |
| 7 | Request Message | Describes a set of messages for requesting information from a detection site. |
| 8 | Device Configuration and Health Status Message | Describes a set of messages for providing information regarding the health status and configuration of devices at a detection site. |
| 9 | Conveyance Tracking Message | Describes a set of messages to provide or request information regarding a conveyance that is the subject of RadNuc detection activities. |
| 10 | System Audit Message | Provides a set of messages for maintaining security audit oversight of the systems/devices utilized in RadNuc detection activities and the data collected by these systems/devices. |

Volumes 3 through 10 provide details of the N.25 message groups. A message group consists of a set of closely related messages that have been grouped into a schema for purposes of efficiently generating, transmitting, receiving, processing, and managing the information exchange.

# Reference Documentation

The following table lists the reference documents for the N.25 IEPD.

Table 3.  Reference Documents

| Document Type | Date | Document Title |
| --- | --- | --- |
| None |  |  |

# N.25 Exchange

## N.25 Purpose

The purpose of N.25 is to provide a standard message exchange protocol for information exchanges in support of the Radiation and Nuclear Detection and Interdiction mission.

## Use Cases

Use cases, when available, are contained in the document volume that addresses the message group that primarily supports the use case.

## Structural Organization of N.25 XML Schemas

The N.25 XML schemas have been designed for conformance with the NIEM standard.  This section explains the schemas that are used and their relationships, as illustrated in Figure 1.  The schemas shown in the four blocks in the Figure 1 are explained in the subsections following the figure.

Figure 2. N.25 XML Schema Relationships

## NIEM Schemas

The following schemas from NIEM are imported by N.25 schema to support references and extensions that are declared therein.  Only the subsetted version of these schema are imported.

Table 4. NIEM Schemas Used by the N.25 IEPD

| Schema | Namespace ID | Description |
| --- | --- | --- |
| niem-core.xsd | nc | The core schema for NIEM containing universal and common components used by all NIEM domains. |
| structures.xsd | s | A supporting NIEM schema containing structures for organizing NIEM components. |
| appinfo.xsd | i | A supporting NIEM schema containing components which provide additional semantics and syntactic guidelines for components built by NIEM-conformant schemas. |
| xsd.xsd | xsd | A supporting NIEM schema that provide Proxy types that carry dictionary metadata and have XML data type simple contents |
| screening.xsd | scr | The schema for the NIEM screening domain. This is principally focused on screening of persons. |
| internationalTrade.xsd | it | The schema for the NIEM international trade domain. |
| intelligence.xsd | Intel | A schema dealing with identification of individuals and information about cautions and/or reasons for which a person might be of interest to an agency. |
| unece-rec20-misc.xsd | unece | A schema providing several standard code lists. |
| nga.xsd | nga | National Geospatial Agency supporting schema. |
| iso_3166.xsd | iso_3166 | Codes for the representation of names of countries and their subdivisions. |
| ansi_d20.xsd | ansi_d20 | .Motor vehicle administration codes |

Note: NIEM contains many other domain schemas; however, they are not identified here since the N.25 and CBRN Domain schema does not use them.

## N.25 Extension Schema

The following comprise the N.25 extension schema.  This is the CBRN Domain schema.

Table 5. Extension Schema Used by the N.25 IEPD

| Schema | Namespace ID | Description |
| --- | --- | --- |
| CBRNdomainCodeLists-v29.xsd | cbrncl | CBRN Code Lists Schema – provides the code lists referenced by the CBRN schema and message schemas. |
| cbrn-e.xsd | cbrn | CBRN Schema – integrated and harmonized schema providing the superset of all elements required by messages/file formats for the CBRN domain. |
| cbrn-ec.xsd | cbrn | Provides the constrained version of the CBRN Domain schema. |

## N.25 Message Exchange Schemas

The N.25 messages required to support radiation detection activities are listed in the following table.

Table 6. N.25 Exchange Schemas

| Schema | Namespace ID | Description |
| --- | --- | --- |
| RadiationDeviceMessage-x.xsd | n25rd | A set of messages to provide information from a radiation detection device. |
| RequestMessage-x.xsd | n25rm | A set of messages to request information from a site. |
| DeviceConfigAndHealthStatusMessage-x.xsd | n25cs | A set of messages to provide configuration and health status data regarding a device. |
| SiteOperationsMessage-x.xsd | n25so | A set of messages to provide information regarding operational activities at a site. |
| SystemAuditMessage-x.xsd | n25sa | A set of messages to provide information regarding the functions and operations of information systems at a detection site. |
| GeneralDeviceMessage-x.xsd | n25gd | A set of messages to provide information from devices used in a radiation detection event, other than a radiation detection device.. |
| ConveyanceTrackingMessage-x.xsd | n25ct | A set of messages to request/provide information regarding a conveyance that has been observed during an encounter. |
| AlarmSummaryMessage-x.xsd | n25as | A message to provide summary information regarding a detection event alarm. |

Note:  In order to distinguish unconstrained and constrained exchange schemas, the convention of providing a suffix has been implemented.  A suffix of “-x” to the file name of a schema identifies it as an unconstrained exchange schema.  A suffix of “-xc” identifies a schema as a constrained exchange schema. In the above table the constraint schemas have not been listed.

## N.25 Exchange Constraint Schemas

Constraint schemas consist of a pair for each message definition schema: one for the message exchange schema and one for the CBRN Domain Schema.  These transform their source definition schemas for use in a software execution environment by applying business rules, such as specifying cardinality, constraining allowable data values via facets, and restricting the choices in enumeration lists.  A constrained exchange schema is identified by the suffix “-xc” to the XSD file name.

# Business Requirements

The basic business requirement for the N.25 exchange messages is to enable standardized and effective reporting of:

- Events and data from deployed radiation and nuclear detection assets (fixed and mobile) as well as corroborative event data to higher level organizations

- Coordination, assessment, and resolution of events

- Site and device configuration and health status

Exchange specific business requirements are contained in the document volume that addresses the message group that primarily supports the business requirement.

# Business Rules

Business rules are contained in the document volume that addresses the message group that must implement the business rules.

# Process Flows and Business Processes

Specific business process models for each message group, when available, are contained in the document volume that addresses the message group that primarily supports the business process.

# Business Scenarios

An typical business scenario may comprise one or more ports of embarcation or ports of enty,  with each port consisting of one or more terminals; each terminal in turn may consist of one or more inspection and screening areas where containers are scanned and inspected by devices and/or people. A location is referred to the inspection and screening area, though the mix of ports and terminals where such an area may be located is expected to vary from country to country. Each location may consist of one or more lanes through which vehicles drive, and each lane may have one or more scanning devices; in some cases, multiple lanes may merge into one or one lane may split into multiple.

Exchange specific business scenarios are contained in the document volume that addresses the message group that supports the scenario.  The following are typical scenarios for the process and data flow at a location where detection events occur.

Scenario 1:  fixed portal

A conveyance (for example, a vehicle or a railcar),  carrying one or more items (e.g., a container or other cargo element), traverses through the available set of scanning devices (RPM, OCR, NII, RIID, and/or VIS). Depending on the venue and device type, some devices may be moving rather than the conveyance/cargo (eg, RIID).

The movement of the conveyance/cargo through the inspection and screening area is referred to as a case, and all data collected during this process is tied together as such using a common case ID.

As the conveyance/cargo passes each device (or the device is applied to the conveyance/cargo), scan data consisting of binary images and/or alphanumeric data are transmitted to the designated local control system. The transmission may not always go directly from the device to the local control system, but may go through an intermediary device-specific software package designed to mediate the interfaces. (Note: depending on the configuration of the detection site, the “local control system” may not be physically local, or collocated with the device(s).)

In addition to the scan data, an analysis of the scan data may be generated and stored for access by local control system either directly from the device or through a device-specific software package.

The local control system stores the scan and analysis data, and implements business logic to determine and flag data that is part of the same case. Of critical importance is maintaining a very high degree of accuracy between the identification of the subject of the scanning activities and the data collected by the devices.

The local control system implements business logic to determine what data set can be sent to other organizations (CBP, DNDO, fusion centers, etc), based on data sharing agreements.  For example, in the case of a detection site at a foreign seaport the typical data sharing agreement is to only allow data transmission to CBP for containers that are destined for the US.

The local control system transmits the permitted scan and analysis data to an organizational element that is responsible for analysis of the data (such as CBP/LSS) to assist in the process of determining the presence of threat materials. The frequency of the transmission is expected to be as immediately as possible after each scan and analysis data package is available.

For containers where the data is ambiguous, additional analysis may be performed offsite and the results provided to the detection site to assist with adjudication.

Scenario 2:

A mobile sensor or a suite of mobile sensors is used to search an area for radiation sources.  In this example, the sensors are deployed through an area rather than having the containers or cargo brought to the sensors.  Searches may be carried out, for example, to look through several city blocks, within the hold of a ship, or at a container storage facility.

Typically sensors will report at discrete intervals and when radiation detections occur they may enter other modes (e.g., identification modes with longer integration times) or request additional detection assets to support detection.  In this example, the data is still integrated into a case, but the case represents the overall search, not a particular vehicle or container.  GPS or other location data is typically collected and associated with the sensor measurements.

As with scenario 1, information may be reported to remote systems for situational awareness, additional analysis, or comparison with historical data.  Results may be transmitted to the searchers to assist them in identifying appropriate courses of action.

Scenario 3:

A mobile sensor is used as an information device for a border patrol agent or law enforcement officer.  In this scenario, the sensor continuously collects data (that may be used to adjust historical/background data).  When an alarm occurs, a remote system may compare with known naturally occurring radioactive material (NORM) sources to reduce false/nuisance alarms.  Alarms that are judged to be real may require additional types of detection (e.g. from slightly different locations or with the operator facing different directions).  The data around the alarm will be associated into a case as will additional detection from additional assets that may be deployed to assist in identification of the alarm.

The preceding scenario examples describe some of the variety in usage of radiation detection systems and the flexibility required of the CBRN Domain model in specifying the data exchanges, but are not intended to be comprehensive.

# Domain Model

The domain model for the N.25 IEPD is the CBRN Domain.  It is described in Volume 2, CBRN Domain.

# Testing and Conformance

Testing of the schemas that comprise the IEPD has been limited by the availability of test tools and environments.  To date the following has occurred.

- The schema are routinely checked for XML well-formedness and validation using Stylus Studio 8.  They validate against W3c and NIEM.

- A subset of the messages has been successfully implemented in a demo project involving Oak Ridge National Lab (DOE) and SPAWAR (US Navy).

Future testing will occur as the IEPD is implemented by CBP/TASPO.

There are currently no available tools to check conformance to NIEM naming rules. However, the rules were applied in development of the IEPD

The implementation of augmentation types cannot at this time be confirmed as NIEM conformant since the NIEM rules do appear to address reuse of augmentation types within the same domain.  At the time of this release this issue continues to be addressed with NIEM.

# Tools and Methodologies

This section provides information regarding the tools and methodologies used to develop and maintain the N.25 IEPD.

## IEPD Definition

To expand NIEM use while enabling the reusability of local implementations, Information Exchange Package Documents (IEPDs) were defined and a methodology for their development created.  To understand what an IEPD is, it is important to understand the foundation upon which it is built.

EXtensible Markup Language (XML) is the first layer in the foundation of IEPDs.  XML offers technologists a universal way of sending and receiving data.  For those not familiar with XML, think of it being much like HTML.  While HTML has tags that define how data is displayed on a website or screen (centered, bold, red, or 12-point font), XML has tags that describe what the data is (victim, agency, automobile, or suspect).  The limitation of XML, however, is that it does not inherently constrain how one can describe data.  That means one could call the Volvo S80 an <automobile>, a second could call it a <make and model>, and another could call it the <get-away vehicle>.  So NIEM was born; a way to constrain XML for all our citizen welfare communities.

Given the vast and widely diverse communities it supports, NIEM contains an enormous number of elements. It is prudent, therefore, to develop Information Exchange Packages (IEPs) which carry only the sub-set of NIEM elements necessary for a specific information exchange purpose. This allows implementers to limit their focus to NIEM elements relevant to the particular exchange events, triggers, conditions, and agencies their IEP is designed to support.  This leads us to the development of Information Exchange Package Documents (IEPDs).

An IEPD is a collection of artifacts that support an implementer’s creation of IEPs.  IEP Documents are typically comprised of seven unique artifacts.  A domain model provides a graphical depiction of the data being exchanged and the cardinality of contained data elements to one another.  IEPDs include a Component Mapping spreadsheet that associates each exchange data element with its corresponding NIEM data element and namespace.  Third, IEPDs include the actual NIEM compliant schemas which can consist of exchange, extension, and constraint schemas.  Since most people are not comfortable reading XML schemas, it is useful to provide schema outlines, sample XML message instance, and associated style-sheets to help practitioners validate the model, mapping, and schemas in a user-friendly way.  The XML Instance is simply the schema with actual data while the style-sheet transforms that Instance into a user-known document, form, or screen. Lastly, the seventh IEPD artifact is a written description of the business scenarios, processes and rules for the data exchange defined by the IEPD.  This collection of IEPD artifacts gives implementers tangible products that can be tailored for local implementation.  Use of IEPDs has proven to save time and money on interface development phases from requirements to testing.  Moreover, use of IEPD artifacts advances the widespread adoption of national standards as well as the realization of reuse benefits.

## Development Process Description

A robust, yet flexible methodology exists for creating and refining IEPDs, graphically illustrated in the following figure.  This process is not unlike most system development life-cycles where requirements are defined, analysis and design performed, and development and testing executed.  The biggest difference with an IEPD development process is the end result is a series of artifacts that can be used in the ultimate system interface development by subsequent implementers.

Excel

Spreadsheet

Metis Visual Object Model

Generated from Metis Model

- WantList

- Subset Schema

- Extension Schema

- Exchange Schema

- Constraint Schema

Generate from Metis model

- Catalog

- Metadata

- Schema outline

- Sample Message template

Figure 3. IEPD Development Process

## N.25 IEPD Approach

Early assessments of the expected domain model with respect to NIEM indicated that less than 20% of the required elements exist in NIEM 2.0.  This is not surprising since the IEPD deals with radiation and nuclear detection events and data, and this has not been previously addressed in NIEM.  Thus it was anticipated that a very large extension schema and multiple exchange schema would need to be designed, developed, and maintained.  Also, given the typically iterative nature of a development project and multiple levels of review, the various artifacts would require frequent updating.

Since there was no existing integrated development environment for the IEPD process and artifacts, the project determined to implement an IEPD tooling environment in a just-in-time manner; functional capabilities were added as needed to support each step in the development process.  The starting point was a commercial object modeling tool that had been previously acquired by DNDO:  Troux Architect (also known as Metis).

The basic concept was to model the IEPD and generate as many of the IEPD artifacts from the model, rather than create them manually using a variety of tools such as Excel, Word, Visio, and XML Spy.  This approach is analygous to the Model Driven Architecture (MDA) approach developed by the Object Modeling Group (OMG). The IEPD master source data model provides:

- Single Source of data

- Continuous enhancement to the single source thru each process step;  each process step “adds value”

- All processes/tools interact with the single source

- Artifacts are generated from the single source

- NIEM rules incorporated into modeling and artifact generation software

- Configuration Management is applied to single source, not artifacts

This approach is illustrated in the following figure:

Configuration

Managed

Continuously Enhance Master Source Data

Process

1

Process

2

Process

3

Process

n

Generate Artifacts/Revisions

Artifact

A

Artifact

B

Artifact

C

Iterate &

Revise

Figure 4. Model Driven IEPD Development Approach

Significant benefits of this approach include:

- Eliminate Process Stovepiping

  - Avoid excessive labor effort caused by:

    - Multiple manual entry for each artifact

    - Changes that have ripple effect thru multiple artifacts

  - Avoid quality impacts from manual entry/update

  - Avoid repetitive manual effort that impacts schedule

  - Reduce cost/schedule/quality risks

- Reduce XML & NIEM level of expertise required – broaden resource pool for IEPD work

- Improve standardization of NIEM conformance

  - Consistency of rules application

The software tooling environment is based on the commercial object modeling tool, Troux Architect, extended to model NIEM IEPDs and generate exchange schemas, exchange constraint schemas, extension schema, extension constraint schema, metadata document, catalog document, message schema outlines, sample message templates, and want list.  The want list is batch input to the GTRI Subset Schema Generator to produce the IEPD Subset Schema. In addition, the NIEM code list generator is also used.

The approach has been successful in producing a very large NIEM domain model with eight message groups comprising 45 messages.  The total number of types, elements, and attributes are listed in the following table.  This includes 105 items obtained from NIEM via the WantList.

Table 7. Count of Types, Elements, and Attributes

| CBRN Domain | 831 |
| --- | --- |
| CodeLists | 93 |
| RadiationDeviceMessage | 27 |
| GeneralDeviceMessage | 56 |
| AlarmSummaryMessage | 59 |
| RequestMessage | 44 |
| DeviceConfig&HealthMessage | 48 |
| SiteOperationsInfoMessage | 164 |
| SystemAuditMessage | 55 |
| ConveyanceTrackingMessage | 40 |
| Total | 1417 |

The IEPD package consists of a large number of artifacts, since there are eight message groups, each with their own set of artifacts, plus the extension schema (CBRN Domain).  The following table lists the number of each type of artifact and whether it is manually created or automatically generated from the master model.

Table 8. N.25 IEPD Artifacts

| Artifact | Manually Created | Auto Generated | Total |
| --- | --- | --- | --- |
| Component Mapping Spreadsheet | 10 |  | 10 |
| Visual Model | 9 |  | 9 |
| Exchange Schema |  | 8 | 8 |
| Constraint Schema |  | 8 | 8 |
| Extension Schema |  | 1 | 1 |
| Extension Constraint Schema |  | 1 | 1 |
| Code List Schema |  | 1 | 1 |
| Want List |  | 1 | 1 |
| Subset Schema |  | 13 | 13 |
| Catalog |  | 1 | 1 |
| Metadata |  | 1 | 1 |
| Schema Outline |  | 8 | 8 |
| Sample Message |  | 8 | 8 |
| Master Document | 10 |  | 10 |
| Total | 29 | 51 | 80 |

# IEPD Metadata

| IEPD Metadata |  |
| --- | --- |
| Metadata | Definition |
| Name | N.25 IEPD Metadata |
| Description | This artifact provides NIEM-conformant metadata about the N.25 IEPD. |
| Summary | The N.25 IEPD provides exchange messages containing data collected by radiation and nuclear detection sites and devices in the operation of the sites, conduct of detection activities, and occurrence of detection events. |
| IEPD Author Name | W. R. Wright |
| IEPD Author Title | Chief Data Architect |
| IEPD Author Component | DHS/DNDO/SESP |
| IEPD Author Phone Number | 425.922.9229 |
| IEPD Author Email Address | bwright@archsmart.net |
| WebSite | none |
| Security | NO CLASSIFICATION |
| Distribution | Distribution statement is TBD |
| CreationDate | 2/14/2009 |
| VersionId | 1.1.40 |
| VersionDate | 2/14/2009 |
| NIEMVersion | 2.0.0 |
| MetadataRevisionDate | 2/14/2009 |
| NextMetadataRevisionDate | unknown |
| Maturity | in review |
| StatusDescription | This IEPD is in review. No testing has occurred other than schema validation. |
| Schedule | TBD |
| Endorsements | None |
| Sponsors | CBP/OIT and DNDO/OCIO |
| Lineage | All elements of this IEPD are original. Use or incorporation of other IEPDs has not occurred. |
| Relationships | This IEPD is not related to any other IEPD. |
| Keywords | radiation, nuclear, device, alarm, conveyance, cargo, encounter, threat |
| Domains | Radiation and Nuclear Detection and Interdiction |
| Purpose | This IEPD provides messages for exchange of data between operational sites that conduct radiation and nuclear threat material detection actvities and organizations that provide analytical, support, fusion, and coordination activities for the RadNuc detection and interdiction mission. |
| MessageExchangePatterns | Request-Response Messages, Asynchronous Messages |
| CommunicationsEnvironment | The N.25 IEPD has been defined independent of any communication transport mechanism/environment. |
| ExchangePartnerCategories | Deployed radnuc detection sites, RadNuc threat materials analysis organizations, Fusion Centers, Emergency Operations Centers, Watch Centers |
| ExchangePartners | Deployed radnuc detection sites, CBP/LSS, DNDO/JAC |
| BusinessProcess | CBRN Detection & Interdiction Encounter |
| TriggeringEvent | There are many triggering events since the IEPD at this version incorporates 50 different messages. For specifics please refer to the IEPD Master Document volumes. |
| Conditions | There are many conditions since the IEPD at this version incorporates 45 different messages. For specifics please refer to the IEPD Master Document volumes. |
| AuthoritativeSourceOrganizationName | CBP/OIT/TASPO and DNDO/OCIO |

In addition to the table above, IEPD metadata is also provided in a machine readable format (XML) as well.  See Appendix B - Catalog.

# IEPD Catalog

This section of the document includes a copy of the IEPD Catalog that has been created in a separate section of the IEPD package.

| IEPD Artifact Catalog |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| IEPD Artifact | Description | Artifact Directory | Artifact File Name | Version ID | Version Date |
| Exchange Files |  |  |  |  |  |
| Subset Schema | Subset of the full NIEM schema used by N.25 and the RadNuc Domain —a compressed directory of schemas to distinguish from other schema sets. | IEPD N.25/Version 1/Version 1.1.40/Schemas/Subset | niem/fbi/2.0/fbi.xsd niem/unece_rec20-misc/2.0/unece_rec20-misc.xsd niem/niem-core/2.0/niem-core.xsd niem/external/w3c-xmlschema/2001/xmlschema.xsd niem/proxy/xsd/2.0/xsd.xsd, niem/structures/2.0/structures.xsd niem/appinfo/2.0/appinfo.xsd niem/domains/screening/2.0/screening.xsd, niem/nga/2.0/nga.xsd niem/iso_3166/2.0/iso_3166.xsd niem/domains/intelligence/2.0/intelligence.xsd niem/domains/internationalTrade/2.0/internationalTrade.xsd niem/ansi_d20/2.0/ansi_d20.xsd niem/domains/emergencyManagement/2.0/emergencyManagement.xsd | 2.0.0 | 2/14/2009 |
| Wantlist | User requirements –identifies the NIEM xsd types requested for use within the NIEM Subset Schema. Generated from the N.25 model and up-loaded to the Schema Subset Generation Tool (SSGT). | IEPD N.25/Version 1/Version 1.1.40/Documentation/Wantlist | N.25_IEPD_Wantlist-v40.xml | 1.1.40 | 2/14/2009 |
| Exchange Schema | Base document schemas for the N.25 messages. Also known as the document schema, reference schema, or root schema. | IEPD N.25/Version 1/Version 1.1.40/Schemas/Exchange | RadiationDeviceMessage-e.xsd DeviceConfigAndHealthStatusMessage-e.xsd RequestMessage-e.xsd AlarmSummaryMessage-e.xsd SiteOperationsInfoMessage-e.xsd ConveyanceTrackingMessage-e.xsd GeneralDeviceMessage-e.xsd SystemAuditMessage-e.xsd | 1.1.40 | 2/14/2009 |
| Constraint Schema | The exchange schemas with constraints included. These are the actual schemas to use for message forming, parsing, and validation. | IEPD N.25/Version 1/Version 1.1.40/Schemas/Exchange | RadiationDeviceMessage-ec.xsd DeviceConfigAndHealthStatusMessage-ec.xsd RequestMessage-ec.xsd AlarmSummaryMessage-ec.xsd SiteOperationsInfoMessage-ec.xsd ConveyanceTrackingMessage-ec.xsd GeneralDeviceMessage-ec.xsd SystemAuditMessage-ec.xsd | 1.1.40 | 2/14/2009 |
| Extension Schema | Specification for extended components—separate local name-space of components not contained in NIEM. At this time the RadNuc Domain schema provides this artifact. | IEPD N.25/Version 1/Version 1.1.40/Schemas/Extension | CBRN-x.xsd CBRN-xc.xsd | 1.1.40 | 2/14/2009 |
| Code List Schema | The schema that contains the code list types and enumerations required by the Exchange and Extension Schema. | IEPD N.25/Version 1/Version 1.1.40/Schemas/Subset | cbrncl-v29.xsd | 1.1.40 | 2/14/2009 |
| Sample XML Instance | Example instance messages. | IEPD N.25/Version 1/Version 1.1.40/Schemas/Sample Messages | RadiationDeviceSampleMessageTemplate-v40.xml DeviceConfigAndHealthStatusSampleMessageTemplate-v40.xml RequestSampleMessageTemplate-v40.xml AlarmSummarySampleMessageTemplate-v40.xml SiteOperationsInfoSampleMessageTemplate-v40.xml ConveyanceTrackingSampleMessageTemplate-v40.xml GeneralDeviceSampleMessageTemplate-v40.xml SystemAuditSampleMessageTemplate-v40.xml | 1.1.40 | 2/14/2009 |
| Style sheet | Example stylesheets for display of instance messages. |  |  |  |  |
| Documentation |  |  |  |  |  |
| Master Documentation | IEPD documentation providing purpose, business requirements, what, when, why, how to, etc. Includes business requirements and rules, methodology and tools, and testing and conformance information. | IEPD N.25/Version 1/Version 1.1.40/Documentation/Master Documents | N25 IEPD Main Document-Volume 1-Overview-v40.doc N25 IEPD Main Document-Volume 2-RadNuc Domain-v40.doc N25 IEPD Main Document-Volume 3- Radiation Alarm Summary Message-v40.doc N25 IEPD Main Document-Volume 4-Radiation Device Message-v40.doc N25 IEPD Main Document-Volume 5-General Device Message-v40.doc N25 IEPD Main Document-Volume 6-Site Operations Information Message-v40.doc N25 IEPD Main Document-Volume 7-Request Message-v40.doc N25 IEPD Main Document-Volume 8-Device Configuration and Health Status Message-v40.doc N25 IEPD Main Document-Volume 9-Conveyance Tracking Message-v40.doc N25 IEPD Main Document-Volume 10-System Audit Message-v40.doc | 1.1.40 | 2/14/2009 |
| MOUs | Memorandums of understanding among participating agencies. | IEPD N.25/Version 1/Version 1.1.40/Documentation/MOU-Endorsements | None |  |  |
| Endorsement letters | Documentation from professional or governmental organizations that confirm support. Refer to Endorsement in metadata. | IEPD N.25/Version 1/Version 1.1.40/Documentation/MOU-Endorsements | None |  |  |
| Domain model | Domain model for N.25 and the CBRN Domain. The model is a visual object model created with the Metis commercial modeling system. | IEPD N.25/Version 1/Version 1.1.40/Documentation/Visual Models | RadNucDomainModel-v40.png DeviceConfigAndHealthStatusMessage-v40.png SystemAuditMessage-v40.png SiteOperationsInfoMessage-v40.png ConveyanceTrackingMessage-v40.png RequestMessage-v40.png GeneralDeviceMessage-v40.png RadAlarmSummaryMessage-v40.png RadiationDeviceMessage-v40.png | 1.1.40 | 2/14/2009 |
| Exchange model | Use case or business process diagrams | Included in Master Documents |  |  |  |
| Mapping to NIEM | Mapping of domain components to NIEM components; tagged with cardinality constraints. Also known as the Component Mapping Template. | IEPD N.25/Version 1/Version 1.1.40/Documentation/Mapping Templates | RadNuc Domain-Mapping-CMT-v40.xls IEPD_N.25-SystemAuditMessage-CMT-v40.xls IEPD_N.25-SiteOperationsInfoMessage-CMT-v40.xls IEPD_N.25-ConveyanceTrackingMessage-CMT-v40.xls IEPD_N.25-RequestMessage-CMT-v40.xls IEPD_N.25-DeviceHealthAndConfigMessage-CMT-v40.xls IEPD_N.25-GeneralDeviceMessage-CMT-v40.xls IEPD_N.25-RadiationDeviceMessage-CMT-v40.xls IEPD_N.25-RadiationAlarmSummaryMessage-CMT-v40.xls | 1.1.40 | 2/14/2009 |
| Change log | Document that records the cumulative changes from previous IEPD versions. | IEPD N.25/Version 1/Version 1.1.40/Documentation/Change Log | N25 iepd change log v39 to v40-14 Feb 09.xls | 1.1.40 | 2/14/2009 |
| Catalog |  |  |  |  |  |
| Catalog | List of artifacts in the IEPD that is machine-readable; in an open, portable format and browser-displayable. | IEPD N.25/Version 1/Version 1.1.40/Catalog | N25_IEPD_Catalog-v40.doc | 1.1.40 | 2/14/2009 |
| Metadata | All metadata registered with the IEPD | IEPD N.25/Version 1/Version 1.1.40/Metadata | N25_IEPD_Metadata-v40.xml | 1.1.40 | 2/14/2009 |
| Package |  |  |  |  |  |
| Package | Zip file containing the schemas and supporting files packaged for easy delivery and installation for use. | IEPD N.25/ | IEPD N.25 Package-Build 40.zip | 1.1.40 | 2/14/2009 |